Ryan Smith

015792504

CMPE 146 - Sec 01

22 September 2024

Lab 2

Exercise 1 Bit-Banding

Exercise 1.1 DriverLib

1.)

The code snippet below is setting the proper LED pins for outputs and the s1 button as an input pin.

//set all 3 pins for each LED color as outputs

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN2);

//set control bit to low for each LED pin

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

//set button s1 as input pin with pull up resistor

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

The snippet below initializes the state of the button and the LED.

//initialize s1 button states and blue LED state

uint8_t s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

uint8_t s1_prev_state = s1_curr_state;

int LED_state = 0;

The code snippet below handles the logic of pressing the button and turning the LED on or off.

//loop forever

while(1)

{

//set the current state of the s1 button

s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

2.) When the button is pressed it will consume more power because it connects VCC directly to ground causing more current to flow through the pull up resistor than when just connected to the input pin.

3.) Exercise1.1.MOV

Exercise 1.2 Direct Port Access

1.) The port address for the blue LED is 0x40004C03 and it utilizes the 2nd bit for control. The port address for the push button is 0x40004C00 and it uses the 1st bit for control.

2.)

//loop forever

while(1)

{

//check the current state of the s1 button

if(*s1_button_ptr & (1 << 1))

{

s1_curr_state = 1;

}

else

{

s1_curr_state = 0;

}

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

*blue_LED_ptr = *blue_LED_ptr | (1 << 2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

*blue_LED_ptr = *blue_LED_ptr & ~(1 << 2);

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

3.) My control loop is similar to exercise 1.1. However, I just used bitwise operations on the values at the address pointed to by the s1_button_ptr and blue_LED_ptr. In the first part of my loop, I check the state of the button to see if it has been pressed or not. I use an AND operation with the value at the button address and 1, with a left bit shift of 1 in order to check if the 1st bit is set or not. If it is set, then that means the button is not pressed and the state is 1. If it is not set, then the button has been pressed and the state is 0. Then I check if the current state has changed from the previous state, from 1 to 0, signifying a button press. Then if the LED state is off, I set the 2nd bit for the LED, and if it is on, then I clear the 2nd bit using OR and NAND operations. I end off my control loop by setting the previous push button state to the current push button state.

Exercise 1.3 Bit-Banding Access

1.)

[CORTEX_M4_0] Blue LED alias address: 0x42098068

Push button alias address: 0x42098004

2.)

//loop forever

while(1)

{

//set the current state of the button using the alias ptr

s1_curr_state = *s1_button_alias;

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high using blue LED alias ptr

if(LED_state == 0)

{

*blue_LED_alias = 1;

LED_state = 1;

}

//if LED is on then set LED pin output to low using blue LED alias ptr

else if(LED_state == 1)

{

*blue_LED_alias = 0;

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

3.) The advantage of this control method is that you don’t need to use bitwise operations to check or manipulate a certain bit stored at the address, since the alias address only points to a specific bit.

Exercise 2 Time Measurement

1.)

[CORTEX_M4_0] 3000000

Time when button is pressed: -7859921

Time when button is released: -13742252

Button press duration: 1960 ms

Time when button is pressed: -17565035

Time when button is released: -18088578

Button press duration: 174 ms

Time when button is pressed: -21120343

Time when button is released: -21491912

Button press duration: 123 ms

Time when button is pressed: -23506333

Time when button is released: -23864674

Button press duration: 119 ms

Time when button is pressed: -26364425

Time when button is released: -74056910

Button press duration: 15897 ms

2.)

//loop forever

while(1)

{

//set the current state of the s1 button

s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//get timer value when button is pressed

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

printf("Time when button is pressed: %d\n", t0);

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 0;

}

else{}

}

//check if button has been released

if(s1_prev_state == 0 && s1_curr_state == 1)

{

//get timer value when button is released

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

printf("Time when button is released: %d\n", t1);

//calculate time in ms using T = 1/f

uint32_t time_in_ms = (t0 - t1) / (3000000 / 1000);

printf("Button press duration: %d ms\n", time_in_ms);

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

Exercise 3 Frequency Measurement

1.)

//time before LED is toggled

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Delay Loop */

for(ii=0;ii<100000;ii++)

{

}

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P1, GPIO_PIN0);

//time after LED is toggled

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//difference in counter values

uint32_t counter_diff = t0 - t1;

printf("counter values difference: %d\n", counter_diff);

//time in ms of the for loop delay using T = 1/F formula

uint32_t time_in_ms = counter_diff / (3000000/1000);

printf("time in ms: %d ms\n", time_in_ms);

//blinking frequency of the LED using F = 1/T formula

float blinking_freq = 1000.0 / (2 * time_in_ms);

printf("blinking frequency: %f\n", blinking_freq);

2.)

[CORTEX_M4_0] 3000000

counter values difference: 1300064

time in ms: 433 ms

blinking frequency: 1.154734

counter values difference: 1300063

time in ms: 433 ms

blinking frequency: 1.154734

counter values difference: 1300064

time in ms: 433 ms

blinking frequency: 1.154734

counter values difference: 1300064

time in ms: 433 ms

blinking frequency: 1.154734

3.) There is some uncertainty in the measurement, from the time it takes to execute the functions that read the timer values, and also the function to toggle the LED on/off. This can add small delays that can affect the measurement of the blinking frequency which depends on the timer values for calculation.

Appendix

Exercise 1

Exercise 1.1 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdio.h>

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set all 3 pins for each LED color as outputs

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN2);

//set control bit to low for each LED pin

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

//set button s1 as input pin with pull up resistor

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

//initialize s1 button states and blue LED state

uint8_t s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

uint8_t s1_prev_state = s1_curr_state;

int LED_state = 0;

//loop forever

while(1)

{

//set the current state of the s1 button

s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

}

Exercise 1.2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdio.h>

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set all 3 pins for each LED color as outputs

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN2);

//set control bit to low for each LED pin

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

//set button s1 as input pin with pull up resistor

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

//initialize s1 button states and blue LED state

uint8_t s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

uint8_t s1_prev_state = s1_curr_state;

int LED_state = 0;

//initialize pointers for blue LED and s1 button

uint8_t *blue_LED_ptr = (uint8_t*)0x40004C03;

uint8_t *s1_button_ptr = (uint8_t*)0x40004C00;

//loop forever

while(1)

{

//check the current state of the s1 button

if(*s1_button_ptr & (1 << 1))

{

s1_curr_state = 1;

}

else

{

s1_curr_state = 0;

}

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

*blue_LED_ptr = *blue_LED_ptr | (1 << 2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

*blue_LED_ptr = *blue_LED_ptr & ~(1 << 2);

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

}

Exercise 1.3 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdio.h>

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set all 3 pins for each LED color as outputs

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN2);

//set control bit to low for each LED pin

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

//set button s1 as input pin with pull up resistor

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

//initialize pointers for blue LED and s1 button

uint8_t *blue_LED_ptr = (uint8_t*)0x40004C03;

uint8_t *s1_button_ptr = (uint8_t*)0x40004C00;

//translate port addresses to alias addresses

uint8_t *blue_LED_alias = (uint8_t*)(((uintptr_t)blue_LED_ptr - 0x40000000) * 32 + 2 * 4 + 0x42000000);

uint8_t *s1_button_alias = (uint8_t*)(((uintptr_t)s1_button_ptr - 0x40000000) * 32 + 1 * 4 + 0x42000000);

//initialize s1 button states and blue LED state

uint8_t s1_curr_state = *s1_button_alias;

uint8_t s1_prev_state = s1_curr_state;

int LED_state = 0;

printf("Blue LED alias address: 0x%x\n", blue_LED_alias);

printf("Push button alias address: 0x%x\n", s1_button_alias);

//loop forever

while(1)

{

//set the current state of the button using the alias ptr

s1_curr_state = *s1_button_alias;

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//if LED is off then set LED pin output to high using blue LED alias ptr

if(LED_state == 0)

{

*blue_LED_alias = 1;

LED_state = 1;

}

//if LED is on then set LED pin output to low using blue LED alias ptr

else if(LED_state == 1)

{

*blue_LED_alias = 0;

LED_state = 0;

}

else{}

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

}

Exercise 2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdio.h>

int main(void)

{

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

printf("%u\n", MAP_CS_getMCLK());

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set all 3 pins for each LED color as outputs

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN2);

//set control bit to low for each LED pin

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

//set button s1 as input pin with pull up resistor

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

//initialize s1 button states and blue LED state

uint8_t s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

uint8_t s1_prev_state = s1_curr_state;

int LED_state = 0;

//initialize timer variables

uint32_t t0 = 0;

uint32_t t1 = 0;

//loop forever

while(1)

{

//set the current state of the s1 button

s1_curr_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

//if the S1 button value changes from 1 -> 0, then button has been pushed and toggle blue LED pin

if(s1_curr_state == 0 && s1_prev_state == 1)

{

//get timer value when button is pressed

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

printf("Time when button is pressed: %d\n", t0);

//if LED is off then set LED pin output to high

if(LED_state == 0)

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 1;

}

//if LED is on then set LED pin output to low

else if(LED_state == 1)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN2);

LED_state = 0;

}

else{}

}

//check if button has been released

if(s1_prev_state == 0 && s1_curr_state == 1)

{

//get timer value when button is released

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

printf("Time when button is released: %d\n", t1);

//calculate time in ms using T = 1/f

uint32_t time_in_ms = (t0 - t1) / (3000000 / 1000);

printf("Button press duration: %d ms\n", time_in_ms);

}

//set previous state to current state so on button release it does not toggle the LED

s1_prev_state = s1_curr_state;

}

}

Exercise 3 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

//![Simple GPIO Config]

int main(void)

{

//initialize timer and print clock speed

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

printf("%u\n", MAP_CS_getMCLK());

volatile uint32_t ii;

/* Halting the Watchdog */

MAP_WDT_A_holdTimer();

/* Configuring P1.0 as output */

MAP_GPIO_setAsOutputPin(GPIO_PORT_P1, GPIO_PIN0);

while (1)

{

//time before LED is toggled

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Delay Loop */

for(ii=0;ii<100000;ii++)

{

}

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P1, GPIO_PIN0);

//time after LED is toggled

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//difference in counter values

uint32_t counter_diff = t0 - t1;

printf("counter values difference: %d\n", counter_diff);

//time in ms of the for loop delay using T = 1/F formula

uint32_t time_in_ms = counter_diff / (3000000/1000);

printf("time in ms: %d ms\n", time_in_ms);

//blinking frequency of the LED using F = 1/T formula

float blinking_freq = 1000.0 / (2 * time_in_ms);

printf("blinking frequency: %f\n", blinking_freq);

}

}

//![Simple GPIO Config]