Ryan Smith

015792504

CMPE 146 - Sec 01

8 October 2024

Lab 3

Exercise 1 CRC-32

Exercise 1.1 Simple Checksum

1.)

//function to create a checksum by adding up all of the bytes in the data array

uint32_t compute_simple_checksum(uint8_t* data, uint32_t length)

{

//initialize checksum and byte_pos to 0

uint32_t checksum = 0;

uint32_t byte_pos = 0;

uint32_t i = 0;

//for each byte in the data array

for(; i < length; i++)

{

//add byte from data array by shifting it one byte or 8 bits times byte_pos left to add it to that specific byte in checksum

checksum += (uint32_t)data[i] << (8 * byte_pos);

//if at highest byte set to 0 to go back to lowest byte

if(byte_pos == 3)

{

byte_pos = 0;

}

//if not at highest byte increment the position

else

{

byte_pos++;

}

}

//reverse 1s to 0s and 0s to 1s in checksum

checksum = ~checksum;

//return the checksum

return checksum;

}

2.)

[CORTEX_M4_0] first 4 values: 0xf 0x62 0x29 0x29

last 4 values: dc 55 39 aa

Checksum: 0xf25b8806

3.)

In order to speed up the function you could create one 32 bit integer from four bytes in the myData array at a time and add that to the checksum. This would decrease the number of iterations in the for loop.

Exercise 1.2 Speedup

1.)

//get timer value right before hardware calculation starts

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_INIT, CRC32_MODE);

for (ii = 0; ii < sizeof(myData); ii++)

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

//print first 4 and last 4 values in myData array to check if the data has been read in correctly from the file

printf("first 4 values: 0x%x 0x%x 0x%x 0x%x\n", myData[0], myData[1], myData[2], myData[3]);

printf("last 4 values: %x %x %x %x\n", myData[length-4], myData[length-3], myData[length-2], myData[length-1]);

/* Getting the result from the hardware module */

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//get timer value once we get result from hardware module

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print hardware calculated checksum

printf("Hardware calculated checksum: 0x%x\n", hwCalculatedCRC);

//calculate time for hardware checksum creation using formula T = 1/F

uint32_t hwTime = (t0 - t1) / (3000000 / 1000000);

printf("Hardware checksum time in microseconds: %d\n", hwTime);

//get timer value right before software calculation starts

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Calculating the CRC32 checksum through software */

swCalculatedCRC = calculateCRC32((uint8_t*) myData, sizeof(myData));

//get timer value when software calculation ends

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print software checksum

printf("Software calculated checksum: 0x%x\n", swCalculatedCRC);

//calculate time for software checksum creation using formula T = 1/F

uint32_t swTime = (t0 - t1) / (3000000 / 1000000);

printf("Software checksum time in microseconds: %d\n", swTime);

//get timer value before computing simple checksum

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//compute simple checksum

uint32_t simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//get timer value after computing simple checksum

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print simple checksum

printf("Simple checksum: 0x%x\n", simple_checksum);

//calculate time for simple checksum computation using formula T = 1/F

uint32_t simpleTime = (t0 - t1) / (3000000 / 1000000);

//print time for simple checksum

printf("Simple checksum time in microseconds: %d\n", simpleTime);

//calculate the speedup as a ratio of time required for the hardware checksum over the time for the software checksum

float speedup = (float)swTime / (float)hwTime;

//print the speedup ratio

printf("Speedup: %f\n", speedup);

2.)

[CORTEX_M4_0] Hardware calculated checksum: 0xd9a716c5

Hardware checksum time in microseconds: 214149

Software calculated checksum: 0xd9a716c5

Software checksum time in microseconds: 832872

Simple checksum: 0xf25b8806

Simple checksum time in microseconds: 107540

Speedup: 3.889217

3.)

Using hardware to calculate the checksum in this case was 3.88 times faster than the software calculated checksum. Two major contributing factors to the speedup are hardwired logic that is faster than using the general processor, and also setting up the accelerator is easy as you just have to pass data to its own registers so that it can then compute using the hardwired logic without running any program.

Exercise 1.3 Simulate data corruption

1.)

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_INIT, CRC32_MODE);

for (ii = 0; ii < sizeof(myData); ii++)

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

//print first 4 and last 4 values in myData array to check if the data has been read in correctly from the file

printf("first 4 values: 0x%x 0x%x 0x%x 0x%x\n", myData[0], myData[1], myData[2], myData[3]);

printf("last 4 values: %x %x %x %x\n", myData[length-4], myData[length-3], myData[length-2], myData[length-1]);

/* Getting the result from the hardware module */

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum

uint32_t simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both original checksums

printf("Original hardware checksum: 0x%x\n", hwCalculatedCRC);

printf("Original simple checksum: 0x%x\n", simple_checksum);

//reverse 7th bit of myData[8000]

myData[8000] = myData[8000] ^ (1 << 7);

//set data in hardware registers to new myData values with one bit changed

for(ii = 0; ii < sizeof(myData); ii++)

{

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

}

//get hardware checksum with one bit changed

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum with one bit changed

simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both checksums with one bit changed

printf("Hardware checksum with one bit changed: 0x%x\n", hwCalculatedCRC);

printf("Simple checksum with one bit changed: 0x%x\n", simple_checksum);

//reverse 7th bit of myData[9016]

myData[9016] = myData[9016] ^ (1 << 7);

//set data in hardware registers to new myData values with two bits changed

for(ii = 0; ii < sizeof(myData); ii++)

{

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

}

//get hardware checksum with two bits changed

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum with two bits changed

simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both checksums with two bits changed

printf("Hardware checksum with two bits changed: 0x%x\n", hwCalculatedCRC);

printf("Simple checksum with two bits changed: 0x%x\n", simple_checksum);

2.)

[CORTEX_M4_0] first 4 values: 0xf 0x62 0x29 0x29

last 4 values: dc 55 39 aa

Original hardware checksum: 0xd9a716c5

Original simple checksum: 0xf25b8806

Hardware checksum with one bit changed: 0xf53ca06c

Simple checksum with one bit changed: 0xf25b8786

Hardware checksum with two bits changed: 0xd36f4a25

Simple checksum with two bits changed: 0xf25b8806

3.) The hardware computed checksums are different between all 3 results. However, the simple computed checksum is the same for both the original and when two bits are changed. It only changed when one bit was changed.

4.) Based on this, the hardware computed method is better because one bit change will produce a different result which is important when verifying the data is correct. If the data changes but the checksum is the same, using the checksum to verify the data is correct is not effective.

Exercise 2 Interrupt

Exercise 2.1 Control

1.) The interrupt method does provide a quicker response because the processor responds to the interrupt as soon as the button is pressed, whereas with polling it may miss the button press on the current iteration of the control loop and have to loop again to check if it has been pressed.

2.) IMG_2007.MOV

Exercise 2.2 Measurement

1.)

[CORTEX_M4_0] Button press duration in ms: 3788	Read index value: 1

Button press duration in ms: 97	Read index value: 3

Button press duration in ms: 82	Read index value: 5

Error: queue is full

Button press duration in ms: 94	Read index value: 7

Button press duration in ms: 75	Read index value: 9

Button press duration in ms: 93	Read index value: 1

Button press duration in ms: 66	Read index value: 3

2.) The queue is long enough for this application because it can store 5 full button presses (pressing the button and releasing it). It is unlikely for the queue to fill up before the control loop can process the records, unless the person is pressing the button as fast as possible.

Exercise 3 DMA

Exercise 3.1 Speedup

1.)

[CORTEX_M4_0] Clock speed: 3000000

Hardware checksum: 0x8b0a5208

DMA checksum: 0x8b0a5208

Hardware calculated checksum time in microseconds: 13007

DMA checksum time in microseconds: 2790

Speedup: 4.662007

Exercise 3.2 Different data block sizes

1.)

[CORTEX_M4_0] Block size: 2

Clock speed: 3000000

Hardware checksum: 0xb7647d

DMA checksum: 0xb7647d

Hardware calculated checksum time in microseconds: 69

DMA checksum time in microseconds: 65

Speedup: 1.061538

Block size: 4

Clock speed: 3000000

Hardware checksum: 0xc704dd7b

DMA checksum: 0xc704dd7b

Hardware calculated checksum time in microseconds: 98

DMA checksum time in microseconds: 70

Speedup: 1.400000

Block size: 16

Clock speed: 3000000

Hardware checksum: 0x552d22c8

DMA checksum: 0x552d22c8

Hardware calculated checksum time in microseconds: 274

DMA checksum time in microseconds: 102

Speedup: 2.686275

Block size: 32

Clock speed: 3000000

Hardware checksum: 0x4a55af67

DMA checksum: 0x4a55af67

Hardware calculated checksum time in microseconds: 508

DMA checksum time in microseconds: 145

Speedup: 3.503448

Block size: 64

Clock speed: 3000000

Hardware checksum: 0x93394e51

DMA checksum: 0x93394e51

Hardware calculated checksum time in microseconds: 978

DMA checksum time in microseconds: 230

Speedup: 4.252174

Block size: 128

Clock speed: 3000000

Hardware checksum: 0x46a0eabc

DMA checksum: 0x46a0eabc

Hardware calculated checksum time in microseconds: 1917

DMA checksum time in microseconds: 401

Speedup: 4.780549

Block size: 256

Clock speed: 3000000

Hardware checksum: 0xe55e964f

DMA checksum: 0xe55e964f

Hardware calculated checksum time in microseconds: 3794

DMA checksum time in microseconds: 742

Speedup: 5.113207

Block size: 786

Clock speed: 3000000

Hardware checksum: 0x30b4c8f3

DMA checksum: 0x30b4c8f3

Hardware calculated checksum time in microseconds: 11567

DMA checksum time in microseconds: 2155

Speedup: 5.367517

Block size: 1024

Clock speed: 3000000

Hardware checksum: 0x8b0a5208

DMA checksum: 0x8b0a5208

Hardware calculated checksum time in microseconds: 15058

DMA checksum time in microseconds: 2790

Speedup: 5.397132

2.)

3.) The speedup increases exponentially for smaller block sizes but levels out for the largest block sizes. However, the speedup is higher for larger blocks of data. The DMA controller is much faster for processing large amounts of data because it can transfer without using the CPU, whereas the CPU has to process the data for the checksum accelerator when not using the DMA controller. For smaller blocks of data, initializing the DMA controller causes there to be minimal speedup because of the time it takes to set up the controller just to process the small amount of data.

Appendix

Exercise 1

Exercise 1.1 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

//function to create a checksum by adding up all of the bytes in the data array

uint32_t compute_simple_checksum(uint8_t* data, uint32_t length)

{

//initialize checksum and byte_pos to 0

uint32_t checksum = 0;

uint32_t byte_pos = 0;

uint32_t i = 0;

//for each byte in the data array

for(; i < length; i++)

{

//add byte from data array by shifting it one byte or 8 bits times byte_pos left to add it to that specific byte in checksum

checksum += (uint32_t)data[i] << (8 * byte_pos);

//if at highest byte set to 0 to go back to lowest byte

if(byte_pos == 3)

{

byte_pos = 0;

}

//if not at highest byte increment the position

else

{

byte_pos++;

}

}

//reverse 1s to 0s and 0s to 1s in checksum

checksum = ~checksum;

//return the checksum

return checksum;

}

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//myData array stores input data in bytes from file

static uint8_t myData[10240];

//length of myData array

int length = sizeof(myData);

//call compute checksum function and store it in checksum variable

uint32_t checksum = compute_simple_checksum(myData, length);

//print first 4 and last 4 values in myData array to check if the data has been read in correctly from the file

printf("first 4 values: 0x%x 0x%x 0x%x 0x%x\n", myData[0], myData[1], myData[2], myData[3]);

printf("last 4 values: %x %x %x %x\n", myData[length-4], myData[length-3], myData[length-2], myData[length-1]);

//print checksum as hex value

printf("Checksum: 0x%x\n", checksum);

return 0;

}

Exercise 1.2 Program

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#include <stdio.h>

#define CRC32_POLY              0xEDB88320

#define CRC32_INIT              0xFFFFFFFF

static uint8_t myData[10240];

static uint32_t calculateCRC32(uint8_t* data, uint32_t length);

volatile uint32_t hwCalculatedCRC, swCalculatedCRC;

//function to create a checksum by adding up all of the bytes in the data array

uint32_t compute_simple_checksum(uint8_t* data, uint32_t length)

{

//initialize checksum and byte_pos to 0

uint32_t checksum = 0;

uint32_t byte_pos = 0;

uint32_t i = 0;

//for each byte in the data array

for(; i < length; i++)

{

//add byte from data array by shifting it one byte or 8 bits times byte_pos left to add it to that specific byte in checksum

checksum += (uint32_t)data[i] << (8 * byte_pos);

//if at highest byte set to 0 to go back to lowest byte

if(byte_pos == 3)

{

byte_pos = 0;

}

//if not at highest byte increment the position

else

{

byte_pos++;

}

}

//reverse 1s to 0s and 0s to 1s in checksum

checksum = ~checksum;

//return the checksum

return checksum;

}

//![Simple CRC32 Example]

int main(void)

{

//initialize timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

//initialize timer variables

uint32_t t0 = 0;

uint32_t t1 = 0;

uint32_t ii;

/* Stop WDT */

MAP_WDT_A_holdTimer();

//get timer value right before hardware calculation starts

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_INIT, CRC32_MODE);

for (ii = 0; ii < sizeof(myData); ii++)

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

//print first 4 and last 4 values in myData array to check if the data has been read in correctly from the file

printf("first 4 values: 0x%x 0x%x 0x%x 0x%x\n", myData[0], myData[1], myData[2], myData[3]);

printf("last 4 values: %x %x %x %x\n", myData[length-4], myData[length-3], myData[length-2], myData[length-1]);

/* Getting the result from the hardware module */

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//get timer value once we get result from hardware module

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print hardware calculated checksum

printf("Hardware calculated checksum: 0x%x\n", hwCalculatedCRC);

//calculate time for hardware checksum creation using formula T = 1/F

uint32_t hwTime = (t0 - t1) / (3000000 / 1000000);

printf("Hardware checksum time in microseconds: %d\n", hwTime);

//get timer value right before software calculation starts

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Calculating the CRC32 checksum through software */

swCalculatedCRC = calculateCRC32((uint8_t*) myData, sizeof(myData));

//get timer value when software calculation ends

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print software checksum

printf("Software calculated checksum: 0x%x\n", swCalculatedCRC);

//calculate time for software checksum creation using formula T = 1/F

uint32_t swTime = (t0 - t1) / (3000000 / 1000000);

printf("Software checksum time in microseconds: %d\n", swTime);

//get timer value before computing simple checksum

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//compute simple checksum

uint32_t simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//get timer value after computing simple checksum

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//print simple checksum

printf("Simple checksum: 0x%x\n", simple_checksum);

//calculate time for simple checksum computation using formula T = 1/F

uint32_t simpleTime = (t0 - t1) / (3000000 / 1000000);

//print time for simple checksum

printf("Simple checksum time in microseconds: %d\n", simpleTime);

//calculate the speedup as a ratio of time required for the hardware checksum over the time for the software checksum

float speedup = (float)swTime / (float)hwTime;

//print the speedup ratio

printf("Speedup: %f\n", speedup);

/* Pause for the debugger */

__no_operation();

}

//![Simple CRC32 Example]

/* Standard software calculation of CRC32 */

static uint32_t calculateCRC32(uint8_t* data, uint32_t length)

{

uint32_t ii, jj, byte, crc, mask;;

crc = 0xFFFFFFFF;

for(ii=0;ii<length;ii++)

{

byte = data[ii];

crc = crc ^ byte;

for (jj = 0; jj < 8; jj++)

{

mask = -(crc & 1);

crc = (crc >> 1) ^ (CRC32_POLY & mask);

}

}

return ~crc;

}

Exercise 1.3 Program

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#include <stdio.h>

#define CRC32_POLY              0xEDB88320

#define CRC32_INIT              0xFFFFFFFF

static uint8_t myData[10240];

static uint32_t calculateCRC32(uint8_t* data, uint32_t length);

volatile uint32_t hwCalculatedCRC, swCalculatedCRC;

//function to create a checksum by adding up all of the bytes in the data array

uint32_t compute_simple_checksum(uint8_t* data, uint32_t length)

{

//initialize checksum and byte_pos to 0

uint32_t checksum = 0;

uint32_t byte_pos = 0;

uint32_t i = 0;

//for each byte in the data array

for(; i < length; i++)

{

//add byte from data array by shifting it one byte or 8 bits times byte_pos left to add it to that specific byte in checksum

checksum += (uint32_t)data[i] << (8 * byte_pos);

//if at highest byte set to 0 to go back to lowest byte

if(byte_pos == 3)

{

byte_pos = 0;

}

//if not at highest byte increment the position

else

{

byte_pos++;

}

}

//reverse 1s to 0s and 0s to 1s in checksum

checksum = ~checksum;

//return the checksum

return checksum;

}

//![Simple CRC32 Example]

int main(void)

{

uint32_t ii;

uint32_t length = sizeof(myData);

/* Stop WDT */

MAP_WDT_A_holdTimer();

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_INIT, CRC32_MODE);

for (ii = 0; ii < sizeof(myData); ii++)

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

//print first 4 and last 4 values in myData array to check if the data has been read in correctly from the file

printf("first 4 values: 0x%x 0x%x 0x%x 0x%x\n", myData[0], myData[1], myData[2], myData[3]);

printf("last 4 values: %x %x %x %x\n", myData[length-4], myData[length-3], myData[length-2], myData[length-1]);

/* Getting the result from the hardware module */

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum

uint32_t simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both original checksums

printf("Original hardware checksum: 0x%x\n", hwCalculatedCRC);

printf("Original simple checksum: 0x%x\n", simple_checksum);

//reverse 7th bit of myData[8000]

myData[8000] = myData[8000] ^ (1 << 7);

//set data in hardware registers to new myData values with one bit changed

for(ii = 0; ii < sizeof(myData); ii++)

{

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

}

//get hardware checksum with one bit changed

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum with one bit changed

simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both checksums with one bit changed

printf("Hardware checksum with one bit changed: 0x%x\n", hwCalculatedCRC);

printf("Simple checksum with one bit changed: 0x%x\n", simple_checksum);

//reverse 7th bit of myData[9016]

myData[9016] = myData[9016] ^ (1 << 7);

//set data in hardware registers to new myData values with two bits changed

for(ii = 0; ii < sizeof(myData); ii++)

{

MAP_CRC32_set8BitData(myData[ii], CRC32_MODE);

}

//get hardware checksum with two bits changed

hwCalculatedCRC = MAP_CRC32_getResultReversed(CRC32_MODE) ^ 0xFFFFFFFF;

//compute simple checksum with two bits changed

simple_checksum = compute_simple_checksum(myData, sizeof(myData));

//print both checksums with two bits changed

printf("Hardware checksum with two bits changed: 0x%x\n", hwCalculatedCRC);

printf("Simple checksum with two bits changed: 0x%x\n", simple_checksum);

/* Pause for the debugger */

__no_operation();

}

//![Simple CRC32 Example]

/* Standard software calculation of CRC32 */

static uint32_t calculateCRC32(uint8_t* data, uint32_t length)

{

uint32_t ii, jj, byte, crc, mask;;

crc = 0xFFFFFFFF;

for(ii=0;ii<length;ii++)

{

byte = data[ii];

crc = crc ^ byte;

for (jj = 0; jj < 8; jj++)

{

mask = -(crc & 1);

crc = (crc >> 1) ^ (CRC32_POLY & mask);

}

}

return ~crc;

}

Exercise 2

Exercise 2.1 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//configuring green LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN1);

/* Configuring P1.1 (button S1) as an input and enabling interrupts */

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

MAP_GPIO_clearInterruptFlag(GPIO_PORT_P1, GPIO_PIN1);

MAP_GPIO_enableInterrupt(GPIO_PORT_P1, GPIO_PIN1);

MAP_Interrupt_enableInterrupt(INT_PORT1);

//generate interrupt from button press

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_HIGH_TO_LOW_TRANSITION);

MAP_Interrupt_enableMaster();

while(1)

{

}

}

/* GPIO ISR */

void PORT1_IRQHandler(void)

{

uint32_t status;

//get status of interrupt

status = MAP_GPIO_getEnabledInterruptStatus(GPIO_PORT_P1);

MAP_GPIO_clearInterruptFlag(GPIO_PORT_P1, status);

/* Toggling the output on the LED */

//check if status is equal to the button pin

if(status & GPIO_PIN1)

{

//if the button is pressed

if(MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1) == 0)

{

//toggle green LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

//cause interrupt on low to high transition (releasing the button)

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_LOW_TO_HIGH_TRANSITION);

}

//if the button is not pressed

else if(MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1) == 1)

{

//toggle green LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

//cause interrupt on high to low transition (pressing the button)

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_HIGH_TO_LOW_TRANSITION);

}

else{}

}

}

Exercise 2.2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define QUEUE_SIZE 10

struct record {

uint32_t timestamp;

uint8_t state;

};

struct record queue[QUEUE_SIZE];

volatile int read_index;

volatile int write_index;

int main(void)

{

//initialize timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//configuring green LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN1);

/* Configuring P1.1 (button S1) as an input and enabling interrupts */

MAP_GPIO_setAsInputPinWithPullUpResistor(GPIO_PORT_P1, GPIO_PIN1);

MAP_GPIO_clearInterruptFlag(GPIO_PORT_P1, GPIO_PIN1);

MAP_GPIO_enableInterrupt(GPIO_PORT_P1, GPIO_PIN1);

MAP_Interrupt_enableInterrupt(INT_PORT1);

//generate interrupt from button press

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_HIGH_TO_LOW_TRANSITION);

MAP_Interrupt_enableMaster();

//set queue indexes to 0

read_index = 0;

write_index = 0;

uint32_t t0 = 0;

while(1)

{

//add dummy delay

__delay_cycles(200 * 3000);

//check if read_index != write_index meaning the queue is not empty

if(read_index != write_index)

{

//set current to the record in the queue at read_index

struct record current = queue[read_index];

//if the state of the record is 0 button is pressed so set t0 to that timestamp

if (current.state == 0)

{

t0 = current.timestamp;

}

//if the state of the record is 1 button is pressed so then calculate difference in timestamps between the current and previous record

else if(current.state == 1)

{

//calculate duration in ms using formula T = 1/F

uint32_t duration =  (t0 - current.timestamp) / (3000000 / 1000);

//print duration and read_index

printf("Button press duration in ms: %d", duration);

printf("\tRead index value: %d\n", read_index);

}

else{}

read_index = (read_index + 1) % QUEUE_SIZE;

}

}

}

/* GPIO ISR */

void PORT1_IRQHandler(void)

{

uint32_t status;

//get status of interrupt

status = MAP_GPIO_getEnabledInterruptStatus(GPIO_PORT_P1);

MAP_GPIO_clearInterruptFlag(GPIO_PORT_P1, status);

//check if queue is full

if(((write_index + 1) % QUEUE_SIZE == read_index) || (read_index == 0 && write_index == QUEUE_SIZE - 1))

{

printf("Error: queue is full\n");

return;

}

/* Toggling the output on the LED */

//check if status is equal to the button pin

if(status & GPIO_PIN1)

{

//get state of button

uint32_t button_state = MAP_GPIO_getInputPinValue(GPIO_PORT_P1, GPIO_PIN1);

//add timestamp and state to queue and increment write index

queue[write_index].timestamp = MAP_Timer32_getValue(TIMER32_0_BASE);

queue[write_index].state = button_state;

write_index = (write_index + 1) % QUEUE_SIZE;

//if the button is pressed

if(button_state == 0)

{

//toggle green LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

//cause interrupt on low to high transition (releasing the button)

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_LOW_TO_HIGH_TRANSITION);

}

//if the button is not pressed

else if(button_state == 1)

{

//toggle green LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

//cause interrupt on high to low transition (pressing the button)

MAP_GPIO_interruptEdgeSelect(GPIO_PORT_P1, GPIO_PIN1, GPIO_HIGH_TO_LOW_TRANSITION);

}

else{}

}

}

Exercise 3

Exercise 3.1 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdio.h>

#include <string.h>

#include <stdbool.h>

/* Statics */

static volatile uint32_t crcSignature;

/* DMA Control Table */

#if defined(__TI_COMPILER_VERSION__)

#pragma DATA_ALIGN(controlTable, 1024)

#elif defined(__IAR_SYSTEMS_ICC__)

#pragma data_alignment=1024

#elif defined(__GNUC__)

__attribute__ ((aligned (1024)))

#elif defined(__CC_ARM)

__align(1024)

#endif

uint8_t controlTable[1024];

#define CRC32_SEED 0xFFFFFFFF

uint8_t data_array[1024];

volatile int dma_done;

int main(void)

{

//initialize timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

/* Halting Watchdog */

MAP_WDT_A_holdTimer();

//initialize variables to hold timestamps

uint32_t t0 = 0;

uint32_t t1 = 0;

//get timer value before hardware checksum calculation

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_SEED, CRC32_MODE);

int ii;

for (ii = 0; ii < sizeof(data_array); ii++)

MAP_CRC32_set8BitData(data_array[ii], CRC32_MODE);

/* Getting the result from the hardware module */

uint32_t hwCalculatedCRC = MAP_CRC32_getResult(CRC32_MODE);

//get timer value after we retrieve hardware calculated checksum

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate duration in microseconds of hardware calculated checksum

printf("Clock speed: %d\n", MAP_CS_getMCLK());

uint32_t hwTime = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

/* Configuring DMA module */

MAP_DMA_enableModule();

MAP_DMA_setControlBase(controlTable);

/* Setting Control Indexes. In this case we will set the source of the

* DMA transfer to our random data array and the destination to the

* CRC32 data in register address*/

MAP_DMA_setChannelControl(UDMA_PRI_SELECT,

UDMA_SIZE_8 | UDMA_SRC_INC_8 | UDMA_DST_INC_NONE | UDMA_ARB_1024);

MAP_DMA_setChannelTransfer(UDMA_PRI_SELECT,

UDMA_MODE_AUTO, data_array,

(void*) (&CRC32->DI32), 1024);

/* Assigning/Enabling Interrupts */

MAP_DMA_assignInterrupt(DMA_INT1, 0);

MAP_Interrupt_enableInterrupt(INT_DMA_INT1);

MAP_Interrupt_enableMaster();

/* Enabling DMA Channel 0 */

MAP_DMA_enableChannel(0);

//set seed for checksum before data is transferred to accelerator via DMA

MAP_CRC32_setSeed(CRC32_SEED, CRC32_MODE);

//set dma_done flag to 0 before transferring data

dma_done = 0;

//get timer value before DMA checksum

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Forcing a software transfer on DMA Channel 0 */

MAP_DMA_requestSoftwareTransfer(0);

//forever loop to check for status of DMA transfer

while(1)

{

//if the dma transfer is done break from the loop

if(dma_done == 1)

{

//get timer value once DMA checksum is done

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

break;

}

}

//calculate time in microseconds for DMA transfer and checksum calculation

uint32_t dmaTime = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

//calculate speedup

float speedup = (float)hwTime / (float)dmaTime;

//print checksums, times, and speedup of the hardware and dma methods

printf("Hardware checksum: 0x%x\n", hwCalculatedCRC);

printf("DMA checksum: 0x%x\n", crcSignature);

printf("Hardware calculated checksum time in microseconds: %d\n", hwTime);

printf("DMA checksum time in microseconds: %d\n", dmaTime);

printf("Speedup: %f\n", speedup);

return 0;

}

/* Completion interrupt for DMA */

void DMA_INT1_IRQHandler(void)

{

MAP_DMA_disableChannel(0);

crcSignature = MAP_CRC32_getResult(CRC32_MODE);

dma_done = 1;

Exercise 3.2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdio.h>

#include <string.h>

#include <stdbool.h>

/* Statics */

static volatile uint32_t crcSignature;

/* DMA Control Table */

#if defined(__TI_COMPILER_VERSION__)

#pragma DATA_ALIGN(controlTable, 1024)

#elif defined(__IAR_SYSTEMS_ICC__)

#pragma data_alignment=1024

#elif defined(__GNUC__)

__attribute__ ((aligned (1024)))

#elif defined(__CC_ARM)

__align(1024)

#endif

uint8_t controlTable[1024];

#define CRC32_SEED 0xFFFFFFFF

uint8_t data_array[1024];

int size_array[] = {2, 4, 16, 32, 64, 128, 256, 786, 1024};

volatile int dma_done;

int main(void)

{

//initialize timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

/* Halting Watchdog */

MAP_WDT_A_holdTimer();

//initialize variables to hold timestamps

uint32_t t0 = 0;

uint32_t t1 = 0;

/* Configuring DMA module */

MAP_DMA_enableModule();

MAP_DMA_setControlBase(controlTable);

/* Assigning/Enabling Interrupts */

MAP_DMA_assignInterrupt(DMA_INT1, 0);

MAP_Interrupt_enableInterrupt(INT_DMA_INT1);

MAP_Interrupt_enableMaster();

//loop for each element in the size_array

int i;

int array_length = sizeof(size_array)/sizeof(size_array[0]);

for(i = 0; i < array_length; i++)

{

printf("Block size: %d\n", size_array[i]);

//get timer value before hardware checksum calculation

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

//start of hardware calculation

MAP_CRC32_setSeed(CRC32_SEED, CRC32_MODE);

int ii;

for (ii = 0; ii < size_array[i]; ii++)

MAP_CRC32_set8BitData(data_array[ii], CRC32_MODE);

/* Getting the result from the hardware module */

uint32_t hwCalculatedCRC = MAP_CRC32_getResult(CRC32_MODE);

//get timer value after we retrieve hardware calculated checksum

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate duration in microseconds of hardware calculated checksum

printf("Clock speed: %d\n", MAP_CS_getMCLK());

uint32_t hwTime = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

/* Setting Control Indexes. In this case we will set the source of the

* DMA transfer to our random data array and the destination to the

* CRC32 data in register address*/

MAP_DMA_setChannelControl(UDMA_PRI_SELECT,

UDMA_SIZE_8 | UDMA_SRC_INC_8 | UDMA_DST_INC_NONE | UDMA_ARB_1024);

MAP_DMA_setChannelTransfer(UDMA_PRI_SELECT,

UDMA_MODE_AUTO, data_array,

(void*) (&CRC32->DI32), size_array[i]);

//set seed for checksum before data is transferred to accelerator via DMA

MAP_CRC32_setSeed(CRC32_SEED, CRC32_MODE);

/* Enabling DMA Channel 0 */

MAP_DMA_enableChannel(0);

//set dma_done flag to 0 before transferring data

dma_done = 0;

//get timer value before DMA checksum

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

/* Forcing a software transfer on DMA Channel 0 */

MAP_DMA_requestSoftwareTransfer(0);

//forever loop to check for status of DMA transfer

while(1)

{

//if the dma transfer is done break from the loop

if(dma_done == 1)

{

//get timer value once DMA checksum is done

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

break;

}

}

//calculate time in microseconds for DMA transfer and checksum calculation

uint32_t dmaTime = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

//calculate speedup

float speedup = (float)hwTime / (float)dmaTime;

//print checksums, times, and speedup of the hardware and dma methods

printf("Hardware checksum: 0x%x\n", hwCalculatedCRC);

printf("DMA checksum: 0x%x\n", crcSignature);

printf("Hardware calculated checksum time in microseconds: %d\n", hwTime);

printf("DMA checksum time in microseconds: %d\n", dmaTime);

printf("Speedup: %f\n\n", speedup);

}

return 0;

}

/* Completion interrupt for DMA */

void DMA_INT1_IRQHandler(void)

{

MAP_DMA_disableChannel(0);

crcSignature = MAP_CRC32_getResult(CRC32_MODE);

dma_done = 1;

}