Ryan Smith

015792504

CMPE 146 - Sec 01

20 October 2024

Lab 4

Exercise 1 Flash memory

Exercise 1.1 String modifications

1.)

[CORTEX_M4_0] string1: 123123

string2: 123123

string1 address: 2d0c

string2 address: 200009e4

string1: 123123

string2: 023123

2.)

Modifying string1 is not possible because it is a string literal which is not supposed to be modified. However, since string2 is an array of characters, it can be modified. The location of these two variables in memory differ, as string1 is located in flash memory, whereas string2 is located in SRAM.

3.)

Based on the output and referencing the datasheet we can see that string1 is stored in flash memory as string1’s address is 0x2d0c and the Flash main memory region corresponds with the address range of 0x0000_0000 to 0x0003_FFFF. Additionally, we can see that string2 is stored in SRAM as string2’s address is 0x200009e4 and the SRAM region corresponds with the address range of 0x2000_0000 to 0x200F_FFFF. In Table 6-38 Memory Map Access Response, the Flash main memory region can have writes ignored if the sector has write protection enabled. This explains why string1 was not modified.

Exercise 1.2 Program flash memory

1.)

bool get_flash_bank_sector(uint32_t mem_address, uint32_t* bank_number, uint32_t* sector_mask)

{

//if the memory address of the string is greater than the end of the flash main memory address range return false

if(mem_address > flash_main_memory_end_address)

{

return false;

}

//if the memory address is less than half the address range set the bank number to 0

if(mem_address < flash_main_memory_middle_address)

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK0;

}

//if the memory address is greater than half the address range set the bank number to 1

else

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK1;

}

//do a mask of the string's memory address by shifting 1 left the memory address divided by 4096 times

//each bank is divided into 32 sectors

if(*bank_number == FLASH_MAIN_MEMORY_SPACE_BANK0)

{

*sector_mask = 1 << ((mem_address - flash_main_memory_start_address) / 4096);

}

else

{

*sector_mask = 1 << ((mem_address - flash_main_memory_middle_address + 1) / 4096);

}

//return true

return true;

}

2.)

//get the bank and sector for strings 1 and 3

get_flash_bank_sector(string1, &string1_bank, &string1_sector);

get_flash_bank_sector(*string3, &string3_bank, &string3_sector);

//unprotect the sectors in order to modify strings 1 and 3

FlashCtl_unprotectSector ((uint_fast8_t)string1_bank, string1_sector);

FlashCtl_unprotectSector ((uint_fast8_t)string3_bank, string3_sector);

//take the first 2 characters from string2 and place them into string1

ROM_FlashCtl_programMemory(string2, string1, 2);

//take the first 2 characters of string2 and place them into string3

ROM_FlashCtl_programMemory(string2, (void*)string3, 2);

3.)

[CORTEX_M4_0] string1: 123123

string2: 123123

string3: 123123

string1 bank: 0x2

string1 sector: 0x100

string2 bank: 0x1

string2 sector: 0x1

string1: 003123

string2: 043123

string3: 003123

4.)

Due to the way flash memory works, the first 2 characters of strings 1 and 2 were changed to 00 instead of 04. Flash memory can only be changed from 1->0 unless the block is erased. When the char 4 tries to be programmed in char 2 spot for string1 and string3, 00110010 (2 in ascii) 00110100 (4 in ascii), it essentially does an AND on the two values which gives you 00110000 (0 in ascii). This is why string1 and string3 end up with 00 instead of 04.

Exercise 2 Non-volatile power-on-reset counter

1.)

In the following code I manipulate the counter by doing a right bit shift of 1 to ‘decrement’ it. My counter value was initialized to 7 in hex or 111 in binary. Doing a right bit shift after each power on reset would produce the following sequence:

111 -> 011

011 -> 001

001 -> 000

//initialize variables to store bank and sector information for each string

uint32_t counter_bank = 0;

uint32_t counter_sector = 0;

//initialize temp counter variable to store value before flashing

uint32_t temp_counter = counter[0] >> 1;

//get the bank and sector for counter

get_flash_bank_sector(counter, &counter_bank, &counter_sector);

//unprotect the sector in order to modify the counter

FlashCtl_unprotectSector ((uint_fast8_t)counter_bank, counter_sector);

//program the counter value into ROM

ROM_FlashCtl_programMemory((void*)&temp_counter, counter, 4);

2.)

In order to do 13 power-on cycles you would have to change the counter value to have 13 bits instead of 3. You would initialize your counter value to 0x1FFF instead of 0x7.

3.)

IMG_2059.MOV

Exercise 3 SRAM function

1.)

[CORTEX_M4_0] Clock frequency: 48000000

SRAM function starting address 0x1000bb9

flash function starting address 0x31d5

SRAM function time in microseconds 8334

flash function time in microseconds 12500

speedup 1.499880

[CORTEX_M4_0] Clock frequency: 3000000

SRAM function starting address 0x1000bb9

flash function starting address 0x3191

SRAM function time in microseconds 133346

flash function time in microseconds 133344

speedup 0.999985

2.)

The results show that at higher clock speeds SRAM is faster than flash memory. This is probably because the processor requires less overhead to access SRAM compared to flash memory. At lower clock speeds, the difference is negligible between the two.

Appendix

Exercise 1 Programs

Exercise 1.1

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

char* string1 = "123123";

char string2[] = "123123";

void main(void)

{

MAP_WDT_A_holdTimer();

printf("string1: %s\n", string1);

printf("string2: %s\n", string2);

printf("string1 address: %p\n", string1);

printf("string2 address: %p\n", string2);

string1[0] = '0';

string2[0] = '0';

printf("string1: %s\n", string1);

printf("string2: %s\n", string2);

}

Exercise 1.2

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define flash_main_memory_start_address 0x00000000

#define flash_main_memory_end_address 0x0003FFFF

char* string1 = "123123";

char string2[] = "123123";

const char string3[1024 * 160] = {"123123"};

uint32_t flash_main_memory_middle_address = (flash_main_memory_end_address + flash_main_memory_start_address) / 2;

bool get_flash_bank_sector(uint32_t mem_address, uint32_t* bank_number,

uint32_t* sector_mask);

void main(void)

{

MAP_WDT_A_holdTimer();

//print all strings before modifying them

printf("string1: %s\n", string1);

printf("string2: %s\n", string2);

printf("string3: %s\n", string3);

//set the first and second characters in string2 to '0' and '4'

string2[0] = '0';

string2[1] = '4';

//initialize variables to store bank and sector information for each string

uint32_t string1_bank = 0;

uint32_t string1_sector = 0;

uint32_t string3_bank = 0;

uint32_t string3_sector = 0;

//get the bank and sector for strings 1 and 3

get_flash_bank_sector(string1, &string1_bank, &string1_sector);

get_flash_bank_sector(*string3, &string3_bank, &string3_sector);

//unprotect the sectors in order to modify strings 1 and 3

FlashCtl_unprotectSector ((uint_fast8_t)string1_bank, string1_sector);

FlashCtl_unprotectSector ((uint_fast8_t)string3_bank, string3_sector);

//take the first 2 characters from string2 and place them into string1

ROM_FlashCtl_programMemory(string2, string1, 2);

//take the first 2 characters of string2 and place them into string3

ROM_FlashCtl_programMemory(string2, (void*)string3, 2);

//print bank and sector numbers for strings 1 and 3

printf("string1 bank: 0x%x\n", string1_bank);

printf("string1 sector: 0x%x\n", string1_sector);

printf("string2 bank: 0x%x\n", string3_bank);

printf("string2 sector: 0x%x\n", string3_sector);

//print all strings after modifying them

printf("string1: %s\n", string1);

printf("string2: %s\n", string2);

printf("string3: %s\n", string3);

}

bool get_flash_bank_sector(uint32_t mem_address, uint32_t* bank_number, uint32_t* sector_mask)

{

//if the memory address of the string is greater than the end of the flash main memory address range return false

if(mem_address > flash_main_memory_end_address)

{

return false;

}

//if the memory address is less than half the address range set the bank number to 0

if(mem_address < flash_main_memory_middle_address)

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK0;

}

//if the memory address is greater than half the address range set the bank number to 1

else

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK1;

}

//do a mask of the string's memory address by shifting 1 left the memory address divided by 4096 times

//each bank is divided into 32 sectors

if(*bank_number == FLASH_MAIN_MEMORY_SPACE_BANK0)

{

*sector_mask = 1 << ((mem_address - flash_main_memory_start_address) / 4096);

}

else

{

*sector_mask = 1 << ((mem_address - flash_main_memory_middle_address + 1) / 4096);

}

//return true

return true;

}

Exercise 2 Program

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define flash_main_memory_start_address 0x00000000

#define flash_main_memory_end_address 0x0003FFFF

const uint32_t counter[1] = {0x7};

uint32_t flash_main_memory_middle_address = (flash_main_memory_end_address + flash_main_memory_start_address) / 2;

bool get_flash_bank_sector(uint32_t mem_address, uint32_t* bank_number,

uint32_t* sector_mask);

void main(void)

MAP_WDT_A_holdTimer();

//set green and red LED as output pin and set both pins to low

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

//check if the counter value has expired (is 0)

if(counter[0] == 0)

{

//turn green LED off and set red LED to on and enter infinite while loop

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN1);

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN0);

while(1)

{

}

}

else

{

//initialize variables to store bank and sector information for each string

uint32_t counter_bank = 0;

uint32_t counter_sector = 0;

//initialize temp counter variable to store value before flashing

uint32_t temp_counter = counter[0] >> 1;

//get the bank and sector for counter

get_flash_bank_sector(counter, &counter_bank, &counter_sector);

//unprotect the sector in order to modify the counter

FlashCtl_unprotectSector ((uint_fast8_t)counter_bank, counter_sector);

//program the counter value into ROM

ROM_FlashCtl_programMemory((void*)&temp_counter, counter, 4);

//toggle output on the green LED

while(1)

{

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

__delay_cycles(3000000);

}

}

}

bool get_flash_bank_sector(uint32_t mem_address, uint32_t* bank_number, uint32_t* sector_mask)

{

//if the memory address of the string is greater than the end of the flash main memory address range return false

if(mem_address > flash_main_memory_end_address)

{

return false;

}

//if the memory address is less than half the address range set the bank number to 0

if(mem_address < flash_main_memory_middle_address)

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK0;

}

//if the memory address is greater than half the address range set the bank number to 1

else

{

*bank_number = FLASH_MAIN_MEMORY_SPACE_BANK1;

}

//do a mask of the string's memory address by shifting 1 left the memory address divided by 4096 times

//each bank is divided into 32 sectors

if(*bank_number == FLASH_MAIN_MEMORY_SPACE_BANK0)

{

*sector_mask = 1 << ((mem_address - flash_main_memory_start_address) / 4096);

}

else

{

*sector_mask = 1 << ((mem_address - flash_main_memory_middle_address + 1) / 4096);

}

//return true

return true;

}

Exercise 3 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#pragma CODE_SECTION(sram_function, ".TI.ramfunc")

void sram_function()

{

register int i;

register int count = 100000;

for (i=0; i<count; i++)

{

}

}

void flash_function()

{

register int i;

register int count = 100000;

for (i=0; i<count; i++)

{

}

}

int main(void)

{

FlashCtl_disableReadBuffering(FLASH_BANK0, FLASH_INSTRUCTION_FETCH);

FlashCtl_disableReadBuffering(FLASH_BANK1, FLASH_INSTRUCTION_FETCH);

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//initialize timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

//initialize variables to hold timestamps

uint32_t t0 = 0;

uint32_t t1 = 0;

//get timer value before running SRAM function

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

sram_function();

//get timer value after running SRAM function

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate run time in microseconds using formula T=1/F

uint32_t SRAM_time = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

//get timer value before running the function normally

t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

flash_function();

//get timer value after running the normal function

t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate run time in microseconds using formula T=1/F

uint32_t flash_time = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

//calculate speedup

float speedup = (float)flash_time / (float)SRAM_time;

printf("Clock frequency: %d\n", MAP_CS_getMCLK());

printf("SRAM function starting address 0x%p\n", (void*)sram_function);

printf("flash function starting address 0x%p\n", (void*)flash_function);

printf("SRAM function time in microseconds %d\n", SRAM_time);

printf("flash function time in microseconds %d\n", flash_time);

printf("speedup %f\n", speedup);

}