Ryan Smith

015792504

CMPE 146 - Sec 01

3 November 2024

Lab 5

Exercise 1 Delay function

Exercise 1.1 Function implementation

1.)

The following delay function takes a variable count which is the number of ms the function should delay. It first gets the value of the timer and stores it in prev. It then calculates the number of clock cycles required to delay the number of ms specified by count. Then it initializes a counter variable to add to on each iteration of the while loop. The while loop checks if the number of delay cycles required is greater than the counter value. In the while loop it gets the timer value and stores it in a variable called current. It then handles the wrap around logic by checking if current is greater than previous, meaning it went to 0xFFFF0000 and wrapped back to 0xFFFFFFFF. If it has wrapped around, then it adds the following to the counter (the difference between the minimum timer value and prev plus the difference between the maximum timer value and current). If it hasn’t wrapped around it adds the difference between the previous and current timer values to counter. Then it sets the previous timer value to the current value. It then loops until the counter is greater than the number of cycles required.

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}

2.)

[CORTEX_M4_0] Target delay time: 5000 ms	Actual delay: 5000132 microseconds	Error between target and actual: 132 microseconds

Target delay time: 2000 ms	Actual delay: 2000077 microseconds	Error between target and actual: 77 microseconds

Target delay time: 1000 ms	Actual delay: 1000070 microseconds	Error between target and actual: 70 microseconds

Target delay time: 50 ms	Actual delay: 50057 microseconds	Error between target and actual: 57 microseconds

Target delay time: 20 ms	Actual delay: 20051 microseconds	Error between target and actual: 51 microseconds

Target delay time: 10 ms	Actual delay: 10041 microseconds	Error between target and actual: 41 microseconds

Target delay time: 5 ms	Actual delay: 5044 microseconds	Error between target and actual: 44 microseconds

Target delay time: 2 ms	Actual delay: 2053 microseconds	Error between target and actual: 53 microseconds

Target delay time: 1 ms	Actual delay: 1046 microseconds	Error between target and actual: 46 microseconds

Target delay time: 0 ms	Actual delay: 144 microseconds	Error between target and actual: 144 microseconds

Exercise 1.2 Timing control

1.)

The delay time used was 1250 ms.

calculate delay time in ms to blink red led at 0.4 Hz

T = 1/F

T = 1/0.4

T = 2.5 secs

T * 1000 = 2500 ms

2500/2 = 1250 ms

divide by 2 because blink is on and off

The following is the control loop. Since a blink is on and off the delay time is split in half between each state. It will toggle the output on the pin and then delay for 1250 ms before toggling the pin again, so each state takes 1250 ms to complete, totalling 2500 ms.

while(1)

{

//toggle output on red LED pin

GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

//delay for time calculated earlier

delay_ms(delay_time);

}

2.)

In order to measure the frequency of the blinking I used a stopwatch. I started the stopwatch when the red LED turned off, and then counted 10 on and off states before stopping the stopwatch.

3.)

| Number of blinks | Time (seconds) | Frequency (Hz) |
| --- | --- | --- |
| 10 | 24.77 | 0.4037141704 |
| 10 | 24.98 | 0.4003202562 |
| 10 | 24.88 | 0.4019292605 |
| Avg Frequency (Hz) |  | 0.4019878957 |
| Error |  | 0.001987895674 |

Exercise 2 GPIO Oscillator

1.)

[CORTEX_M4_0] 0101010010101010101010100101010101010101011110101010101010101010100101010101010101010101001010101010

Sampling rate: 102284.343750

2.)

First, I initalize an array to store the oscillator output of size SAMPLES which I defined as a macro. I get the timer value before and after the data collection happens. For the data collection, I have a for loop which loops from 0 to SAMPLES, and I store the state of the oscillator in each iteration. After, I calculate the time difference between the timer values, and I use that to calculate the time in seconds of the data collection. Once that is calculated, I divide the number of samples (SAMPLES) by the time in seconds calculated to get the sampling rate. I then print the sampling rate, and the oscillator output.

//initialize array to store oscillator output

bool oscillator_output[SAMPLES];

//get timer value before data collection

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

int i;

for(i = 0; i < SAMPLES; i++)

{

//store oscillator state in output array

oscillator_output[i] = CAPTIO0CTL & 0x200;;

}

//get timer value after data collection

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate difference between timer values

uint32_t time_diff = t0 - t1;

//calculate time in seconds

float time_in_s = (float)time_diff / MAP_CS_getMCLK();

//calculate sampling rate

float sampling_rate = (float)SAMPLES / time_in_s;

//print oscillator output

for(i = 0; i < SAMPLES; i++)

{

printf("%d", oscillator_output[i]);

}

printf("\nSampling rate: %f\n", sampling_rate);

Exercise 3 Frequency Measurement

1.)

Output not touching pin:

[CORTEX_M4_0] Frequency: 6009666.500000

Time: 0.003421 seconds	 Frequency: 6011333.500000 Hz

Time: 0.057002 seconds	 Frequency: 6011000.000000 Hz

Time: 0.141931 seconds	 Frequency: 6010333.500000 Hz

Time: 0.195841 seconds	 Frequency: 6011666.500000 Hz

Time: 0.249771 seconds	 Frequency: 6010666.500000 Hz

Time: 0.301510 seconds	 Frequency: 6011666.500000 Hz

Time: 0.351731 seconds	 Frequency: 6012333.500000 Hz

Time: 0.400425 seconds	 Frequency: 6012000.000000 Hz

Time: 0.450799 seconds	 Frequency: 6011333.500000 Hz

Time: 0.499724 seconds	 Frequency: 6011333.500000 Hz

Time: 0.549874 seconds	 Frequency: 6012333.500000 Hz

Time: 0.599972 seconds	 Frequency: 6011333.500000 Hz

Time: 0.651813 seconds	 Frequency: 6011333.500000 Hz

Time: 0.701310 seconds	 Frequency: 6012666.500000 Hz

Time: 0.753588 seconds	 Frequency: 6011000.000000 Hz

Time: 0.804000 seconds	 Frequency: 6012666.500000 Hz

Time: 0.854332 seconds	 Frequency: 6013000.000000 Hz

Time: 0.905203 seconds	 Frequency: 6011333.500000 Hz

Time: 0.955990 seconds	 Frequency: 6012000.000000 Hz

Time: 1.005164 seconds	 Frequency: 6012000.000000 Hz

Time: 1.055264 seconds	 Frequency: 6012000.000000 Hz

Time: 1.105272 seconds	 Frequency: 6011666.500000 Hz

Time: 1.156471 seconds	 Frequency: 6012333.500000 Hz

Time: 1.205925 seconds	 Frequency: 6012000.000000 Hz

Time: 1.256084 seconds	 Frequency: 6012666.500000 Hz

Time: 1.305416 seconds	 Frequency: 6013000.000000 Hz

Time: 1.355779 seconds	 Frequency: 6011333.500000 Hz

Time: 1.406624 seconds	 Frequency: 6011333.500000 Hz

Time: 1.458082 seconds	 Frequency: 6011666.500000 Hz

Time: 1.510422 seconds	 Frequency: 6010333.500000 Hz

Time: 1.560862 seconds	 Frequency: 6010666.500000 Hz

Time: 1.613005 seconds	 Frequency: 6010000.000000 Hz

Time: 1.664194 seconds	 Frequency: 6010000.000000 Hz

Time: 1.714315 seconds	 Frequency: 6009333.500000 Hz

Time: 1.766236 seconds	 Frequency: 6010000.000000 Hz

Time: 1.816896 seconds	 Frequency: 6010666.500000 Hz

Time: 1.868380 seconds	 Frequency: 6011000.000000 Hz

Time: 1.920106 seconds	 Frequency: 6010666.500000 Hz

Time: 1.972010 seconds	 Frequency: 6011000.000000 Hz

Time: 2.023037 seconds	 Frequency: 6010666.500000 Hz

Time: 2.074439 seconds	 Frequency: 6011000.000000 Hz

Time: 2.124004 seconds	 Frequency: 6010666.500000 Hz

Time: 2.174078 seconds	 Frequency: 6011000.000000 Hz

Time: 2.223317 seconds	 Frequency: 6011333.500000 Hz

Time: 2.273251 seconds	 Frequency: 6010333.500000 Hz

Time: 2.323142 seconds	 Frequency: 6011000.000000 Hz

Time: 2.374016 seconds	 Frequency: 6010666.500000 Hz

Time: 2.449459 seconds	 Frequency: 6011000.000000 Hz

Time: 2.500508 seconds	 Frequency: 6010000.000000 Hz

Time: 2.576814 seconds	 Frequency: 6011000.000000 Hz

Time: 2.671212 seconds	 Frequency: 6011000.000000 Hz

Time: 2.722796 seconds	 Frequency: 6010666.500000 Hz

Time: 2.773200 seconds	 Frequency: 6011666.500000 Hz

Time: 2.824873 seconds	 Frequency: 6012000.000000 Hz

Time: 2.874917 seconds	 Frequency: 6011000.000000 Hz

Time: 2.926154 seconds	 Frequency: 6011666.500000 Hz

Time: 2.977110 seconds	 Frequency: 6011000.000000 Hz

Time: 3.026102 seconds	 Frequency: 6010666.500000 Hz

Time: 3.077358 seconds	 Frequency: 6010666.500000 Hz

Time: 3.128490 seconds	 Frequency: 6011333.500000 Hz

Time: 3.177981 seconds	 Frequency: 6011666.500000 Hz

Time: 3.229435 seconds	 Frequency: 6011666.500000 Hz

Time: 3.280257 seconds	 Frequency: 6012000.000000 Hz

Time: 3.329830 seconds	 Frequency: 6011666.500000 Hz

Time: 3.380870 seconds	 Frequency: 6011000.000000 Hz

Time: 3.431355 seconds	 Frequency: 6012000.000000 Hz

Time: 3.482304 seconds	 Frequency: 6012333.500000 Hz

Time: 3.533598 seconds	 Frequency: 6011666.500000 Hz

Time: 3.585917 seconds	 Frequency: 6011333.500000 Hz

Time: 3.636306 seconds	 Frequency: 6011000.000000 Hz

Time: 3.687528 seconds	 Frequency: 6010666.500000 Hz

Time: 3.737646 seconds	 Frequency: 6011000.000000 Hz

Time: 3.788320 seconds	 Frequency: 6011000.000000 Hz

Time: 3.837429 seconds	 Frequency: 6011666.500000 Hz

Time: 3.887090 seconds	 Frequency: 6012000.000000 Hz

Time: 3.936003 seconds	 Frequency: 6011666.500000 Hz

Time: 3.985879 seconds	 Frequency: 6011666.500000 Hz

Time: 4.036263 seconds	 Frequency: 6012000.000000 Hz

Time: 4.086300 seconds	 Frequency: 6011666.500000 Hz

Time: 4.136636 seconds	 Frequency: 6011666.500000 Hz

Time: 4.186849 seconds	 Frequency: 6012000.000000 Hz

Time: 4.235970 seconds	 Frequency: 6012666.500000 Hz

Time: 4.285404 seconds	 Frequency: 6012333.500000 Hz

Time: 4.335252 seconds	 Frequency: 6011666.500000 Hz

Time: 4.387017 seconds	 Frequency: 6012333.500000 Hz

Time: 4.438036 seconds	 Frequency: 6012000.000000 Hz

Time: 4.488461 seconds	 Frequency: 6010666.500000 Hz

Time: 4.539697 seconds	 Frequency: 6010666.500000 Hz

Time: 4.590410 seconds	 Frequency: 6011333.500000 Hz

Time: 4.640670 seconds	 Frequency: 6011000.000000 Hz

Time: 4.690314 seconds	 Frequency: 6010666.500000 Hz

Time: 4.740127 seconds	 Frequency: 6012333.500000 Hz

Time: 4.791188 seconds	 Frequency: 6011333.500000 Hz

Time: 4.841113 seconds	 Frequency: 6011666.500000 Hz

Time: 4.890416 seconds	 Frequency: 6011333.500000 Hz

Time: 4.940611 seconds	 Frequency: 6010666.500000 Hz

Time: 4.992756 seconds	 Frequency: 6011000.000000 Hz

Time: 5.044187 seconds	 Frequency: 6009666.500000 Hz

Time: 5.094343 seconds	 Frequency: 6010666.500000 Hz

Time: 5.178319 seconds	 Frequency: 6010666.500000 Hz

Time: 5.229030 seconds	 Frequency: 6010000.000000 Hz

Time: 5.279070 seconds	 Frequency: 6009666.500000 Hz

Time: 5.328852 seconds	 Frequency: 6010000.000000 Hz

Time: 5.378077 seconds	 Frequency: 6010333.500000 Hz

Time: 5.427379 seconds	 Frequency: 6011333.500000 Hz

Time: 5.478453 seconds	 Frequency: 6009666.500000 Hz

Time: 5.530024 seconds	 Frequency: 6009666.500000 Hz

Time: 5.580116 seconds	 Frequency: 6011000.000000 Hz

Time: 5.629642 seconds	 Frequency: 6010666.500000 Hz

Time: 5.679574 seconds	 Frequency: 6011000.000000 Hz

Time: 5.730640 seconds	 Frequency: 6010666.500000 Hz

Time: 5.781345 seconds	 Frequency: 6010666.500000 Hz

Time: 5.830737 seconds	 Frequency: 6010333.500000 Hz

Time: 5.880097 seconds	 Frequency: 6010666.500000 Hz

Time: 5.929905 seconds	 Frequency: 6011000.000000 Hz

Time: 5.980123 seconds	 Frequency: 6011333.500000 Hz

Time: 6.030815 seconds	 Frequency: 6010666.500000 Hz

Time: 6.080995 seconds	 Frequency: 6010666.500000 Hz

Time: 6.130869 seconds	 Frequency: 6010666.500000 Hz

Time: 6.181507 seconds	 Frequency: 6010333.500000 Hz

Time: 6.231462 seconds	 Frequency: 6011333.500000 Hz

Time: 6.281276 seconds	 Frequency: 6012000.000000 Hz

Time: 6.330800 seconds	 Frequency: 6011333.500000 Hz

Time: 6.380891 seconds	 Frequency: 6011666.500000 Hz

Time: 6.430579 seconds	 Frequency: 6011333.500000 Hz

Time: 6.480372 seconds	 Frequency: 6011666.500000 Hz

Time: 6.530940 seconds	 Frequency: 6011666.500000 Hz

Time: 6.583011 seconds	 Frequency: 6010333.500000 Hz

Time: 6.633296 seconds	 Frequency: 6011000.000000 Hz

Time: 6.683088 seconds	 Frequency: 6011333.500000 Hz

Time: 6.732718 seconds	 Frequency: 6011333.500000 Hz

Time: 6.782468 seconds	 Frequency: 6011333.500000 Hz

Time: 6.832784 seconds	 Frequency: 6011666.500000 Hz

Time: 6.882484 seconds	 Frequency: 6011666.500000 Hz

Time: 6.933293 seconds	 Frequency: 6011666.500000 Hz

Time: 6.982917 seconds	 Frequency: 6011666.500000 Hz

Time: 7.033289 seconds	 Frequency: 6011333.500000 Hz

Time: 7.083255 seconds	 Frequency: 6011333.500000 Hz

Time: 7.133672 seconds	 Frequency: 6011333.500000 Hz

Time: 7.183445 seconds	 Frequency: 6011666.500000 Hz

Time: 7.234563 seconds	 Frequency: 6011333.500000 Hz

Time: 7.285660 seconds	 Frequency: 6011333.500000 Hz

Time: 7.335020 seconds	 Frequency: 6011333.500000 Hz

Time: 7.385046 seconds	 Frequency: 6012000.000000 Hz

Time: 7.436736 seconds	 Frequency: 6011000.000000 Hz

Time: 7.487325 seconds	 Frequency: 6010666.500000 Hz

Time: 7.537766 seconds	 Frequency: 6010666.500000 Hz

Time: 7.587328 seconds	 Frequency: 6010666.500000 Hz

Time: 7.639344 seconds	 Frequency: 6010666.500000 Hz

Time: 7.690037 seconds	 Frequency: 6010666.500000 Hz

Time: 7.740023 seconds	 Frequency: 6010666.500000 Hz

Time: 7.789561 seconds	 Frequency: 6011333.500000 Hz

Time: 7.840563 seconds	 Frequency: 6011000.000000 Hz

Time: 7.890941 seconds	 Frequency: 6011666.500000 Hz

Time: 7.940829 seconds	 Frequency: 6011000.000000 Hz

Time: 7.991144 seconds	 Frequency: 6010333.500000 Hz

Time: 8.040750 seconds	 Frequency: 6011000.000000 Hz

Time: 8.090362 seconds	 Frequency: 6011333.500000 Hz

Time: 8.141073 seconds	 Frequency: 6011333.500000 Hz

Time: 8.190746 seconds	 Frequency: 6012333.500000 Hz

Time: 8.241436 seconds	 Frequency: 6010333.500000 Hz

Time: 8.291376 seconds	 Frequency: 6010333.500000 Hz

Time: 8.342568 seconds	 Frequency: 6011000.000000 Hz

Time: 8.392845 seconds	 Frequency: 6010333.500000 Hz

Time: 8.444459 seconds	 Frequency: 6010333.500000 Hz

Time: 8.494263 seconds	 Frequency: 6010666.500000 Hz

Time: 8.544150 seconds	 Frequency: 6009333.500000 Hz

Time: 8.594905 seconds	 Frequency: 6010666.500000 Hz

Time: 8.645407 seconds	 Frequency: 6010666.500000 Hz

Time: 8.696770 seconds	 Frequency: 6010000.000000 Hz

Time: 8.747277 seconds	 Frequency: 6011666.500000 Hz

Time: 8.796888 seconds	 Frequency: 6011000.000000 Hz

Time: 8.846998 seconds	 Frequency: 6010000.000000 Hz

Time: 8.897017 seconds	 Frequency: 6011000.000000 Hz

Time: 8.948781 seconds	 Frequency: 6011666.500000 Hz

Time: 8.997923 seconds	 Frequency: 6010666.500000 Hz

Time: 9.048672 seconds	 Frequency: 6010666.500000 Hz

Time: 9.098447 seconds	 Frequency: 6011000.000000 Hz

Time: 9.149035 seconds	 Frequency: 6010333.500000 Hz

Time: 9.198400 seconds	 Frequency: 6010333.500000 Hz

Time: 9.248986 seconds	 Frequency: 6010000.000000 Hz

Time: 9.300574 seconds	 Frequency: 6011666.500000 Hz

Time: 9.352834 seconds	 Frequency: 6011666.500000 Hz

Time: 9.402053 seconds	 Frequency: 6011666.500000 Hz

Time: 9.452522 seconds	 Frequency: 6011333.500000 Hz

Time: 9.501955 seconds	 Frequency: 6011000.000000 Hz

Time: 9.552909 seconds	 Frequency: 6011666.500000 Hz

Time: 9.603371 seconds	 Frequency: 6012000.000000 Hz

Time: 9.653648 seconds	 Frequency: 6011666.500000 Hz

Time: 9.704477 seconds	 Frequency: 6011333.500000 Hz

Time: 9.755076 seconds	 Frequency: 6012000.000000 Hz

Time: 9.804519 seconds	 Frequency: 6011666.500000 Hz

Time: 9.857205 seconds	 Frequency: 6011666.500000 Hz

Time: 9.907504 seconds	 Frequency: 6012333.500000 Hz

Time: 9.958570 seconds	 Frequency: 6011666.500000 Hz

Time: 10.009288 seconds	 Frequency: 6012000.000000 Hz

Output while touching pin:
[CORTEX_M4_0] Frequency: 6021333.500000

Time: 0.003419 seconds	 Frequency: 6023333.500000 Hz

Time: 0.056603 seconds	 Frequency: 6025000.000000 Hz

Time: 0.109633 seconds	 Frequency: 6024333.500000 Hz

Time: 0.192439 seconds	 Frequency: 6024333.500000 Hz

Time: 0.243576 seconds	 Frequency: 6025000.000000 Hz

Time: 0.292955 seconds	 Frequency: 6025000.000000 Hz

Time: 0.341508 seconds	 Frequency: 6024000.000000 Hz

Time: 0.390446 seconds	 Frequency: 6024000.000000 Hz

Time: 0.439725 seconds	 Frequency: 6024666.500000 Hz

Time: 0.489438 seconds	 Frequency: 6025333.500000 Hz

Time: 0.539255 seconds	 Frequency: 6024666.500000 Hz

Time: 0.589449 seconds	 Frequency: 6025000.000000 Hz

Time: 0.638765 seconds	 Frequency: 6025000.000000 Hz

Time: 0.688947 seconds	 Frequency: 6024666.500000 Hz

Time: 0.740397 seconds	 Frequency: 6023666.500000 Hz

Time: 0.790009 seconds	 Frequency: 6024333.500000 Hz

Time: 0.840496 seconds	 Frequency: 6024000.000000 Hz

Time: 0.890583 seconds	 Frequency: 6022333.500000 Hz

Time: 0.940215 seconds	 Frequency: 1381333.375000 Hz

Time: 0.989411 seconds	 Frequency: 1184000.000000 Hz

Time: 1.039654 seconds	 Frequency: 1180000.000000 Hz

Time: 1.089309 seconds	 Frequency: 1179666.625000 Hz

Time: 1.139396 seconds	 Frequency: 1170666.625000 Hz

Time: 1.189778 seconds	 Frequency: 1150333.375000 Hz

Time: 1.240786 seconds	 Frequency: 1157000.000000 Hz

Time: 1.292164 seconds	 Frequency: 1181333.375000 Hz

Time: 1.341726 seconds	 Frequency: 6023666.500000 Hz

Time: 1.391558 seconds	 Frequency: 6024666.500000 Hz

Time: 1.441043 seconds	 Frequency: 6023666.500000 Hz

Time: 1.491370 seconds	 Frequency: 6024666.500000 Hz

Time: 1.543011 seconds	 Frequency: 6024333.500000 Hz

Time: 1.593791 seconds	 Frequency: 6024666.500000 Hz

Time: 1.644128 seconds	 Frequency: 6024333.500000 Hz

Time: 1.694661 seconds	 Frequency: 6025000.000000 Hz

Time: 1.745108 seconds	 Frequency: 6024000.000000 Hz

Time: 1.795861 seconds	 Frequency: 6025000.000000 Hz

Time: 1.845589 seconds	 Frequency: 6025333.500000 Hz

Time: 1.895277 seconds	 Frequency: 6024666.500000 Hz

Time: 1.944981 seconds	 Frequency: 6025000.000000 Hz

Time: 1.994514 seconds	 Frequency: 6024333.500000 Hz

Time: 2.046162 seconds	 Frequency: 6024666.500000 Hz

Time: 2.099613 seconds	 Frequency: 6024333.500000 Hz

Time: 2.150238 seconds	 Frequency: 6004000.000000 Hz

Time: 2.199931 seconds	 Frequency: 921666.687500 Hz

Time: 2.250325 seconds	 Frequency: 888333.312500 Hz

Time: 2.300174 seconds	 Frequency: 905666.687500 Hz

Time: 2.350310 seconds	 Frequency: 1020666.687500 Hz

Time: 2.401050 seconds	 Frequency: 882666.687500 Hz

Time: 2.451398 seconds	 Frequency: 785000.000000 Hz

Time: 2.501937 seconds	 Frequency: 965000.000000 Hz

Time: 2.552697 seconds	 Frequency: 903333.312500 Hz

Time: 2.603348 seconds	 Frequency: 6022000.000000 Hz

Time: 2.653732 seconds	 Frequency: 6025333.500000 Hz

Time: 2.704721 seconds	 Frequency: 6025333.500000 Hz

Time: 2.755423 seconds	 Frequency: 6025000.000000 Hz

Time: 2.804165 seconds	 Frequency: 6024333.500000 Hz

Time: 2.853896 seconds	 Frequency: 6024333.500000 Hz

Time: 2.904999 seconds	 Frequency: 6024000.000000 Hz

Time: 2.955891 seconds	 Frequency: 6024333.500000 Hz

Time: 3.006872 seconds	 Frequency: 6023333.500000 Hz

Time: 3.058407 seconds	 Frequency: 6024000.000000 Hz

Time: 3.108749 seconds	 Frequency: 6023666.500000 Hz

Time: 3.160886 seconds	 Frequency: 6023666.500000 Hz

Time: 3.211757 seconds	 Frequency: 6025000.000000 Hz

Time: 3.262937 seconds	 Frequency: 6025000.000000 Hz

Time: 3.313709 seconds	 Frequency: 984333.312500 Hz

Time: 3.365122 seconds	 Frequency: 813000.000000 Hz

Time: 3.415275 seconds	 Frequency: 954666.687500 Hz

Time: 3.466957 seconds	 Frequency: 961000.000000 Hz

Time: 3.516931 seconds	 Frequency: 935666.687500 Hz

Time: 3.568493 seconds	 Frequency: 789000.000000 Hz

Time: 3.619090 seconds	 Frequency: 832000.000000 Hz

Time: 3.669137 seconds	 Frequency: 800000.000000 Hz

Time: 3.720383 seconds	 Frequency: 799666.687500 Hz

Time: 3.771396 seconds	 Frequency: 1021000.000000 Hz

Time: 3.823449 seconds	 Frequency: 6025333.500000 Hz

Time: 3.874921 seconds	 Frequency: 6025000.000000 Hz

Time: 3.925845 seconds	 Frequency: 6024666.500000 Hz

Time: 3.976550 seconds	 Frequency: 6024666.500000 Hz

Time: 4.026812 seconds	 Frequency: 6024333.500000 Hz

Time: 4.076898 seconds	 Frequency: 6025000.000000 Hz

Time: 4.127063 seconds	 Frequency: 6024333.500000 Hz

Time: 4.177362 seconds	 Frequency: 6025000.000000 Hz

Time: 4.227955 seconds	 Frequency: 6025000.000000 Hz

Time: 4.277328 seconds	 Frequency: 6025000.000000 Hz

Time: 4.327478 seconds	 Frequency: 6025333.500000 Hz

Time: 4.378672 seconds	 Frequency: 6025333.500000 Hz

Time: 4.430752 seconds	 Frequency: 6025333.500000 Hz

Time: 4.481791 seconds	 Frequency: 6025333.500000 Hz

Time: 4.532757 seconds	 Frequency: 6025000.000000 Hz

Time: 4.581895 seconds	 Frequency: 1209333.375000 Hz

Time: 4.632767 seconds	 Frequency: 855333.312500 Hz

Time: 4.683441 seconds	 Frequency: 849666.687500 Hz

Time: 4.734407 seconds	 Frequency: 862333.312500 Hz

Time: 4.783218 seconds	 Frequency: 867666.687500 Hz

Time: 4.833608 seconds	 Frequency: 856666.687500 Hz

Time: 4.883726 seconds	 Frequency: 825666.687500 Hz

Time: 4.933277 seconds	 Frequency: 800333.312500 Hz

Time: 4.984200 seconds	 Frequency: 850333.312500 Hz

Time: 5.034008 seconds	 Frequency: 873000.000000 Hz

Time: 5.084976 seconds	 Frequency: 874000.000000 Hz

Time: 5.135261 seconds	 Frequency: 6024000.000000 Hz

Time: 5.186772 seconds	 Frequency: 6025000.000000 Hz

Time: 5.238821 seconds	 Frequency: 6024666.500000 Hz

Time: 5.288494 seconds	 Frequency: 6024666.500000 Hz

Time: 5.338841 seconds	 Frequency: 6024666.500000 Hz

Time: 5.389853 seconds	 Frequency: 6024666.500000 Hz

Time: 5.441103 seconds	 Frequency: 6024666.500000 Hz

Time: 5.491056 seconds	 Frequency: 6025000.000000 Hz

Time: 5.540966 seconds	 Frequency: 6024666.500000 Hz

Time: 5.591697 seconds	 Frequency: 6025000.000000 Hz

Time: 5.642895 seconds	 Frequency: 6025000.000000 Hz

Time: 5.693515 seconds	 Frequency: 6025000.000000 Hz

Time: 5.744066 seconds	 Frequency: 6024333.500000 Hz

Time: 5.793931 seconds	 Frequency: 6024333.500000 Hz

Time: 5.843357 seconds	 Frequency: 6025333.500000 Hz

Time: 5.894387 seconds	 Frequency: 6024666.500000 Hz

Time: 5.945114 seconds	 Frequency: 6023666.500000 Hz

Time: 5.996206 seconds	 Frequency: 6024333.500000 Hz

Time: 6.048306 seconds	 Frequency: 6024000.000000 Hz

Time: 6.099293 seconds	 Frequency: 6023666.500000 Hz

Time: 6.150365 seconds	 Frequency: 6023666.500000 Hz

Time: 6.200332 seconds	 Frequency: 6024333.500000 Hz

Time: 6.252068 seconds	 Frequency: 6024666.500000 Hz

Time: 6.301971 seconds	 Frequency: 6024333.500000 Hz

Time: 6.352304 seconds	 Frequency: 6024333.500000 Hz

Time: 6.401636 seconds	 Frequency: 6024000.000000 Hz

Time: 6.451639 seconds	 Frequency: 6024333.500000 Hz

Time: 6.502079 seconds	 Frequency: 6023666.500000 Hz

Time: 6.551675 seconds	 Frequency: 6024333.500000 Hz

Time: 6.602254 seconds	 Frequency: 6024333.500000 Hz

Time: 6.651444 seconds	 Frequency: 6024666.500000 Hz

Time: 6.701869 seconds	 Frequency: 6023666.500000 Hz

Time: 6.752912 seconds	 Frequency: 6024666.500000 Hz

Time: 6.804666 seconds	 Frequency: 6024000.000000 Hz

Time: 6.853909 seconds	 Frequency: 6024666.500000 Hz

Time: 6.904403 seconds	 Frequency: 6024333.500000 Hz

Time: 6.954703 seconds	 Frequency: 6023666.500000 Hz

Time: 7.004557 seconds	 Frequency: 6024000.000000 Hz

Time: 7.053967 seconds	 Frequency: 6024000.000000 Hz

Time: 7.105561 seconds	 Frequency: 6024333.500000 Hz

Time: 7.155839 seconds	 Frequency: 6024333.500000 Hz

Time: 7.206394 seconds	 Frequency: 6023000.000000 Hz

Time: 7.256158 seconds	 Frequency: 6023000.000000 Hz

Time: 7.306222 seconds	 Frequency: 6023000.000000 Hz

Time: 7.357098 seconds	 Frequency: 6023000.000000 Hz

Time: 7.406717 seconds	 Frequency: 6023333.500000 Hz

Time: 7.456972 seconds	 Frequency: 6022333.500000 Hz

Time: 7.506143 seconds	 Frequency: 6021666.500000 Hz

Time: 7.556370 seconds	 Frequency: 6021666.500000 Hz

Time: 7.606889 seconds	 Frequency: 6021333.500000 Hz

Time: 7.657519 seconds	 Frequency: 6018666.500000 Hz

Time: 7.708456 seconds	 Frequency: 6008333.500000 Hz

Time: 7.757772 seconds	 Frequency: 1207333.375000 Hz

Time: 7.810240 seconds	 Frequency: 1178333.375000 Hz

Time: 7.860903 seconds	 Frequency: 1176666.625000 Hz

Time: 7.910427 seconds	 Frequency: 1139333.375000 Hz

Time: 7.961327 seconds	 Frequency: 1122333.375000 Hz

Time: 8.012732 seconds	 Frequency: 1039000.000000 Hz

Time: 8.062490 seconds	 Frequency: 1067666.625000 Hz

Time: 8.111875 seconds	 Frequency: 999333.312500 Hz

Time: 8.161564 seconds	 Frequency: 6022666.500000 Hz

Time: 8.212841 seconds	 Frequency: 6023666.500000 Hz

Time: 8.263665 seconds	 Frequency: 6024333.500000 Hz

Time: 8.314163 seconds	 Frequency: 6024333.500000 Hz

Time: 8.363817 seconds	 Frequency: 6024000.000000 Hz

Time: 8.414019 seconds	 Frequency: 6024666.500000 Hz

Time: 8.464309 seconds	 Frequency: 6025000.000000 Hz

Time: 8.514688 seconds	 Frequency: 6024333.500000 Hz

Time: 8.565474 seconds	 Frequency: 6024666.500000 Hz

Time: 8.616610 seconds	 Frequency: 6025333.500000 Hz

Time: 8.666607 seconds	 Frequency: 6024333.500000 Hz

Time: 8.715366 seconds	 Frequency: 6024333.500000 Hz

Time: 8.765614 seconds	 Frequency: 6024666.500000 Hz

Time: 8.817288 seconds	 Frequency: 6023666.500000 Hz

Time: 8.867467 seconds	 Frequency: 6024666.500000 Hz

Time: 8.918278 seconds	 Frequency: 6025666.500000 Hz

Time: 8.968518 seconds	 Frequency: 6024666.500000 Hz

Time: 9.021608 seconds	 Frequency: 6025333.500000 Hz

Time: 9.069637 seconds	 Frequency: 6025333.500000 Hz

Time: 9.120687 seconds	 Frequency: 6025000.000000 Hz

Time: 9.170175 seconds	 Frequency: 6026000.000000 Hz

Time: 9.220345 seconds	 Frequency: 6024666.500000 Hz

Time: 9.269691 seconds	 Frequency: 6025333.500000 Hz

Time: 9.319674 seconds	 Frequency: 6025333.500000 Hz

Time: 9.371339 seconds	 Frequency: 6024666.500000 Hz

Time: 9.423047 seconds	 Frequency: 6024000.000000 Hz

Time: 9.472326 seconds	 Frequency: 6025000.000000 Hz

Time: 9.522297 seconds	 Frequency: 6024333.500000 Hz

Time: 9.572421 seconds	 Frequency: 6024666.500000 Hz

Time: 9.622629 seconds	 Frequency: 1007000.000000 Hz

Time: 9.672482 seconds	 Frequency: 783333.312500 Hz

Time: 9.722942 seconds	 Frequency: 889666.687500 Hz

Time: 9.773578 seconds	 Frequency: 844666.687500 Hz

Time: 9.824049 seconds	 Frequency: 1011333.312500 Hz

Time: 9.874582 seconds	 Frequency: 6024333.500000 Hz

Time: 9.925138 seconds	 Frequency: 6025333.500000 Hz

Time: 9.974929 seconds	 Frequency: 6024000.000000 Hz

Time: 10.023656 seconds	 Frequency: 6024333.500000 Hz

2.)

First, I clear the counter, then I delay for 3 ms. I then get the number of counter ticks after the delay. Using the number of ticks, I calculate the frequency in Hz by dividing the number of ticks by (3 ms/1000 ms) to get the number of ticks per second.

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

float freq = (float)counter / (3.0/1000.0);

3.)

4.)

To make the frequency change slightly, I hovered near the pin instead of touching it. In order to change it drastically, I would touch the pin. Since hovering near the pin adds less capacitance than physically touching the pin, the frequency change is less severe.

Exercise 4 LED control

1.)

In the following code I take 3 frequency measurements by clearing the counter, delaying 3 ms, and then taking the count value. For each count value I calculate the frequency. I then find the average of the 3 frequencies I calculated and multiply that by 0.9, so that it does not accidentally get triggered, to set the threshold frequency value. In the control loop below, I calculate the frequency, and compare it to the threshold. If it is less than the threshold, the pin has been pressed, and the LED is toggled on or off.

//create array to store frequencies

float frequencies[3];

//for loop to get control parameters

int i;

for(i = 0; i < 3; i++)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//store frequencies

frequencies[i] = (float)counter / (3.0/1000.0);

}

//calculate average frequency for threshold and multiply by 0.9 to make sure it does not accidentally trigger LED

float threshold = (frequencies[0] + frequencies[1] + frequencies[2]) / 3;

threshold = threshold * 0.9;

//set prev state and led state

bool prev_state = false;

bool led_state = false;

bool curr_state = false;

while(1)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//calculate frequency

float freq = (float)counter / (3.0/1000.0);

//set the current state to see if the button has been pressed or not and set the current state accordingly

if(freq < threshold)

{

curr_state = true;

}

else

{

curr_state = false;

}

//if the prev state is 1 and the current state is 0 that means the button has been pressed and unpressed so toggle LED

if(prev_state && !curr_state)

{

//if LED is on turn LED off

if(led_state)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

led_state = false;

}

//if LED is off turn LED on

else

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN0);

led_state = true;

}

}

//set the previous state equal to the current state

prev_state = curr_state;

}

2.)

[CORTEX_M4_0] Frequency threshold: 5419600.000000 Hz

3.)

IMG_2111.MOV

Appendix

Exercise 1

Exercise 1.1 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

//define min and max timer values

#define MAX_TIMER_VAL 0xFFFFFFFF

#define MIN_TIMER_VAL 0xFFFF0000

//initialize array of delay values

const uint32_t delay_values[] = {5000, 2000, 1000, 50, 20, 10, 5, 2, 1, 0};

void delay_ms(uint32_t count);

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set up timer with 16 bit capacity

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_16BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

//set up 2nd timer to get time measurements for delay values

MAP_Timer32_initModule(TIMER32_1_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_1_BASE, 0);

//loop through all delay values

int i;

for(i = 0; i < sizeof(delay_values)/sizeof(delay_values[0]); i++)

{

uint32_t t0 = MAP_Timer32_getValue(TIMER32_1_BASE);

delay_ms(delay_values[i]);

uint32_t t1 = MAP_Timer32_getValue(TIMER32_1_BASE);

uint32_t actual_delay = (t0 - t1) / (MAP_CS_getMCLK() / 1000000);

uint32_t error = actual_delay - (delay_values[i] * 1000);

printf("Target delay time: %d ms", delay_values[i]);

printf("\tActual delay: %d microseconds", actual_delay);

printf("\tError between target and actual: %d microseconds\n", error);

}

return 0;

}

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}

Exercise 1.2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

//define min and max timer values

#define MAX_TIMER_VAL 0xFFFFFFFF

#define MIN_TIMER_VAL 0xFFFF0000

//initialize array of delay values

const uint32_t delay_values[] = {5000, 2000, 1000, 50, 20, 10, 5, 2, 1, 0};

void delay_ms(uint32_t count);

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//set up timer with 16 bit capacity

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_16BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

//set up 2nd timer to get time measurements for delay values

MAP_Timer32_initModule(TIMER32_1_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT,

TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_1_BASE, 0);

//set up red LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

//calculate delay time in ms to blink red led at 0.4 Hz

//T = 1/F

//T = 1/0.4

//T = 2.5 secs

//T * 1000 = 2500 ms

//divide by 2 because blink is on and off

uint32_t delay_time = 1250;

while(1)

{

//toggle output on red LED pin

GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

//delay for time calculated earlier

delay_ms(delay_time);

}

}

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}

Exercise 2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define SAMPLES 100

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

CAPTIO0CTL = 0; // Clear control register

CAPTIO0CTL |= 0x0100; // Enable CAPTIO

CAPTIO0CTL |= 4 << 4; // Select Port 4. Place port number in Bits 7-4

CAPTIO0CTL |= 1 << 1; // Select Pin 1. Place pin number in Bits 3-1

//initialize array to store oscillator output

bool oscillator_output[SAMPLES];

//get timer value before data collection

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

int i;

for(i = 0; i < SAMPLES; i++)

{

//store oscillator state in output array

oscillator_output[i] = CAPTIO0CTL & 0x200;;

}

//get timer value after data collection

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate difference between timer values

uint32_t time_diff = t0 - t1;

//calculate time in seconds

float time_in_s = (float)time_diff / MAP_CS_getMCLK();

//calculate sampling rate

float sampling_rate = (float)SAMPLES / time_in_s;

//print oscillator output

for(i = 0; i < SAMPLES; i++)

{

printf("%d", oscillator_output[i]);

}

printf("\nSampling rate: %f\n", sampling_rate);

}

Exercise 3 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define SAMPLES 100

//define min and max timer values

#define MAX_TIMER_VAL 0xFFFFFFFF

#define MIN_TIMER_VAL 0xFFFF0000

void delay_ms(uint32_t count);

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//initialize 32 bit timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

CAPTIO0CTL = 0; // Clear control register

CAPTIO0CTL |= 0x0100; // Enable CAPTIO

CAPTIO0CTL |= 4 << 4; // Select Port 4. Place port number in Bits 7-4

CAPTIO0CTL |= 1 << 1; // Select Pin 1. Place pin number in Bits 3-1

Timer_A_ContinuousModeConfig timer_continuous_obj;

timer_continuous_obj.clockSource = TIMER_A_CLOCKSOURCE_INVERTED_EXTERNAL_TXCLK;

timer_continuous_obj.clockSourceDivider = TIMER_A_CLOCKSOURCE_DIVIDER_1;

timer_continuous_obj.timerInterruptEnable_TAIE =

TIMER_A_TAIE_INTERRUPT_DISABLE;

timer_continuous_obj.timerClear = TIMER_A_DO_CLEAR;

MAP_Timer_A_configureContinuousMode(TIMER_A2_BASE, &timer_continuous_obj);

MAP_Timer_A_startCounter(TIMER_A2_BASE, TIMER_A_CONTINUOUS_MODE);

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

float freq = (float)counter / (3.0/1000.0);

printf("Frequency: %f\n", freq);

//get clock speed

uint32_t clk = MAP_CS_getMCLK();

//calculate how many cycles is in 10 seconds to use in while loop

uint32_t ten_secs = clk * 10;

//get timer value at start of measurement loop

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

while(1)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//calculate frequency

freq = (float)counter / (3.0/1000.0);

//get timer value at end of measurement

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

float time_in_s = (t0 - t1) / (float)clk;

printf("Time: %f seconds\t Frequency: %f Hz\n", time_in_s, freq);

fflush(stdout);

//check if time elapsed is greater than 10 seconds

if((t0 - t1) >= ten_secs)

{

break;

}

}

while(1);

}

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}

Exercise 4 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define SAMPLES 100

//define min and max timer values

#define MAX_TIMER_VAL 0xFFFFFFFF

#define MIN_TIMER_VAL 0xFFFF0000

void delay_ms(uint32_t count);

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//initialize 32 bit timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

CAPTIO0CTL = 0; // Clear control register

CAPTIO0CTL |= 0x0100; // Enable CAPTIO

CAPTIO0CTL |= 4 << 4; // Select Port 4. Place port number in Bits 7-4

CAPTIO0CTL |= 1 << 1; // Select Pin 1. Place pin number in Bits 3-1

Timer_A_ContinuousModeConfig timer_continuous_obj;

timer_continuous_obj.clockSource = TIMER_A_CLOCKSOURCE_INVERTED_EXTERNAL_TXCLK;

timer_continuous_obj.clockSourceDivider = TIMER_A_CLOCKSOURCE_DIVIDER_1;

timer_continuous_obj.timerInterruptEnable_TAIE =

TIMER_A_TAIE_INTERRUPT_DISABLE;

timer_continuous_obj.timerClear = TIMER_A_DO_CLEAR;

MAP_Timer_A_configureContinuousMode(TIMER_A2_BASE, &timer_continuous_obj);

MAP_Timer_A_startCounter(TIMER_A2_BASE, TIMER_A_CONTINUOUS_MODE);

//set up red LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

//create array to store frequencies

float frequencies[3];

//for loop to get control parameters

int i;

for(i = 0; i < 3; i++)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//store frequencies

frequencies[i] = (float)counter / (3.0/1000.0);

}

//calculate average frequency for threshold and multiply by 0.9 to make sure it does not accidentally trigger LED

float threshold = (frequencies[0] + frequencies[1] + frequencies[2]) / 3;

threshold = threshold * 0.9;

printf("Frequency threshold: %f Hz\n", threshold);

//get clock speed

uint32_t clk = MAP_CS_getMCLK();

//set prev state and led state

bool prev_state = false;

bool led_state = false;

bool curr_state = false;

while(1)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//calculate frequency

float freq = (float)counter / (3.0/1000.0);

//set the current state to see if the button has been pressed or not and set the current state accordingly

if(freq < threshold)

{

curr_state = true;

}

else

{

curr_state = false;

}

//if the prev state is 1 and the current state is 0 that means the button has been pressed and unpressed so toggle LED

if(prev_state && !curr_state)

{

//if LED is on turn LED off

if(led_state)

{

MAP_GPIO_setOutputLowOnPin(GPIO_PORT_P2, GPIO_PIN0);

led_state = false;

}

//if LED is off turn LED on

else

{

MAP_GPIO_setOutputHighOnPin(GPIO_PORT_P2, GPIO_PIN0);

led_state = true;

}

}

//set the previous state equal to the current state

prev_state = curr_state;

}

}

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}