Ryan Smith

015792504

CMPE 146 Sec 01

1 December 2024

Lab 7

Exercise 1 ADC

Exercise 1.1 Digitize - software trigger

1.)

[CORTEX_M4_0] MCLK: 3000000

10271	Voltage: 752.27 mV

10267	Voltage: 751.98 mV

10279	Voltage: 752.86 mV

10281	Voltage: 753.00 mV

10260	Voltage: 751.46 mV

10276	Voltage: 752.64 mV

10276	Voltage: 752.64 mV

2.)

If we lower the resolution to 12 bits, the digitized values will decrease and the voltages will stay the same because there are only 4096 instead of 16384 possible levels. Since there are less levels, the resolution of the voltage will be less, but it will still result in a similar voltage.

Exercise 1.2 Digitize - timer trigger

1.)

[CORTEX_M4_0] Measured sampling frequency: 99.90 Hz

2.)

[CORTEX_M4_0] Measured sampling frequency: 99.90 Hz

9168	Voltage: 671.48 mV

9092	Voltage: 665.92 mV

9091	Voltage: 665.84 mV

9087	Voltage: 665.55 mV

9096	Voltage: 666.21 mV

9088	Voltage: 665.63 mV

Exercise 1.3 Temperature

1.)

[CORTEX_M4_0] MCLK: 3000000

Calibration values: 30 C: 10375, 85 C: 11857

Before touching the MCU:

10211	Voltage: 747.88 mV	Temperature: 75.04 F

10208	Voltage: 747.66 mV	Temperature: 74.84 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

10218	Voltage: 748.39 mV	Temperature: 75.51 F

10206	Voltage: 747.51 mV	Temperature: 74.71 F

10217	Voltage: 748.32 mV	Temperature: 75.45 F

10226	Voltage: 748.97 mV	Temperature: 76.05 F

10227	Voltage: 749.05 mV	Temperature: 76.11 F

10203	Voltage: 747.29 mV	Temperature: 74.51 F

10218	Voltage: 748.39 mV	Temperature: 75.51 F

10221	Voltage: 748.61 mV	Temperature: 75.71 F

10227	Voltage: 749.05 mV	Temperature: 76.11 F

10217	Voltage: 748.32 mV	Temperature: 75.45 F

10222	Voltage: 748.68 mV	Temperature: 75.78 F

10228	Voltage: 749.12 mV	Temperature: 76.18 F

10215	Voltage: 748.17 mV	Temperature: 75.31 F

10215	Voltage: 748.17 mV	Temperature: 75.31 F

10216	Voltage: 748.24 mV	Temperature: 75.38 F

10216	Voltage: 748.24 mV	Temperature: 75.38 F

10212	Voltage: 747.95 mV	Temperature: 75.11 F

10218	Voltage: 748.39 mV	Temperature: 75.51 F

10220	Voltage: 748.54 mV	Temperature: 75.65 F

10208	Voltage: 747.66 mV	Temperature: 74.84 F

10225	Voltage: 748.90 mV	Temperature: 75.98 F

10227	Voltage: 749.05 mV	Temperature: 76.11 F

10222	Voltage: 748.68 mV	Temperature: 75.78 F

10223	Voltage: 748.75 mV	Temperature: 75.85 F

10216	Voltage: 748.24 mV	Temperature: 75.38 F

10222	Voltage: 748.68 mV	Temperature: 75.78 F

10219	Voltage: 748.46 mV	Temperature: 75.58 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

10212	Voltage: 747.95 mV	Temperature: 75.11 F

10221	Voltage: 748.61 mV	Temperature: 75.71 F

10210	Voltage: 747.80 mV	Temperature: 74.98 F

10218	Voltage: 748.39 mV	Temperature: 75.51 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

10212	Voltage: 747.95 mV	Temperature: 75.11 F

10215	Voltage: 748.17 mV	Temperature: 75.31 F

10216	Voltage: 748.24 mV	Temperature: 75.38 F

10219	Voltage: 748.46 mV	Temperature: 75.58 F

10210	Voltage: 747.80 mV	Temperature: 74.98 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

10212	Voltage: 747.95 mV	Temperature: 75.11 F

10203	Voltage: 747.29 mV	Temperature: 74.51 F

10221	Voltage: 748.61 mV	Temperature: 75.71 F

10215	Voltage: 748.17 mV	Temperature: 75.31 F

10213	Voltage: 748.02 mV	Temperature: 75.18 F

10227	Voltage: 749.05 mV	Temperature: 76.11 F

10217	Voltage: 748.32 mV	Temperature: 75.45 F

10219	Voltage: 748.46 mV	Temperature: 75.58 F

After touching the MCU:

10235	Voltage: 749.63 mV	Temperature: 76.65 F

10240	Voltage: 750.00 mV	Temperature: 76.98 F

10231	Voltage: 749.34 mV	Temperature: 76.38 F

10233	Voltage: 749.49 mV	Temperature: 76.51 F

10237	Voltage: 749.78 mV	Temperature: 76.78 F

10235	Voltage: 749.63 mV	Temperature: 76.65 F

10240	Voltage: 750.00 mV	Temperature: 76.98 F

10232	Voltage: 749.41 mV	Temperature: 76.45 F

10244	Voltage: 750.29 mV	Temperature: 77.25 F

10235	Voltage: 749.63 mV	Temperature: 76.65 F

10231	Voltage: 749.34 mV	Temperature: 76.38 F

10233	Voltage: 749.49 mV	Temperature: 76.51 F

10231	Voltage: 749.34 mV	Temperature: 76.38 F

10235	Voltage: 749.63 mV	Temperature: 76.65 F

10224	Voltage: 748.83 mV	Temperature: 75.91 F

10226	Voltage: 748.97 mV	Temperature: 76.05 F

10232	Voltage: 749.41 mV	Temperature: 76.45 F

10227	Voltage: 749.05 mV	Temperature: 76.11 F

10236	Voltage: 749.71 mV	Temperature: 76.71 F

10235	Voltage: 749.63 mV	Temperature: 76.65 F

10217	Voltage: 748.32 mV	Temperature: 75.45 F

10218	Voltage: 748.39 mV	Temperature: 75.51 F

10229	Voltage: 749.19 mV	Temperature: 76.25 F

10223	Voltage: 748.75 mV	Temperature: 75.85 F

10221	Voltage: 748.61 mV	Temperature: 75.71 F

10213	Voltage: 748.02 mV	Temperature: 75.18 F

10225	Voltage: 748.90 mV	Temperature: 75.98 F

10237	Voltage: 749.78 mV	Temperature: 76.78 F

10242	Voltage: 750.15 mV	Temperature: 77.12 F

10236	Voltage: 749.71 mV	Temperature: 76.71 F

10230	Voltage: 749.27 mV	Temperature: 76.31 F

10232	Voltage: 749.41 mV	Temperature: 76.45 F

10213	Voltage: 748.02 mV	Temperature: 75.18 F

10244	Voltage: 750.29 mV	Temperature: 77.25 F

10225	Voltage: 748.90 mV	Temperature: 75.98 F

10231	Voltage: 749.34 mV	Temperature: 76.38 F

10230	Voltage: 749.27 mV	Temperature: 76.31 F

10240	Voltage: 750.00 mV	Temperature: 76.98 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

10234	Voltage: 749.56 mV	Temperature: 76.58 F

10252	Voltage: 750.88 mV	Temperature: 77.78 F

10214	Voltage: 748.10 mV	Temperature: 75.24 F

2.)

As the temperature of the MCU increases, the frequency decreases as shown in the following output. Before touching the MCU, the frequency is around 601000, but after touching the MCU, the frequency drops below 600000.

[CORTEX_M4_0] Frequency: 6021000.000000

Time: 0.003421 seconds	 Frequency: 6018666.500000 Hz

Time: 0.059558 seconds	 Frequency: 6017333.500000 Hz

Time: 0.113324 seconds	 Frequency: 6016000.000000 Hz

Time: 0.190730 seconds	 Frequency: 6013666.500000 Hz

Time: 0.241141 seconds	 Frequency: 6013333.500000 Hz

Time: 0.291554 seconds	 Frequency: 6013666.500000 Hz

Time: 0.340865 seconds	 Frequency: 6013666.500000 Hz

Time: 0.389624 seconds	 Frequency: 6013000.000000 Hz

Time: 0.440765 seconds	 Frequency: 6013666.500000 Hz

Time: 0.490878 seconds	 Frequency: 6013000.000000 Hz

Time: 0.541143 seconds	 Frequency: 6012333.500000 Hz

Time: 0.590748 seconds	 Frequency: 6013333.500000 Hz

Time: 0.640254 seconds	 Frequency: 6013333.500000 Hz

Time: 0.691053 seconds	 Frequency: 6012333.500000 Hz

Time: 0.741096 seconds	 Frequency: 6011000.000000 Hz

Time: 0.790162 seconds	 Frequency: 6012000.000000 Hz

Time: 0.840898 seconds	 Frequency: 6010666.500000 Hz

Time: 0.892006 seconds	 Frequency: 6011333.500000 Hz

Time: 0.942742 seconds	 Frequency: 6010666.500000 Hz

Time: 0.992632 seconds	 Frequency: 6011000.000000 Hz

Time: 1.043697 seconds	 Frequency: 6011000.000000 Hz

Time: 1.093807 seconds	 Frequency: 6012000.000000 Hz

Time: 1.144520 seconds	 Frequency: 6011000.000000 Hz

Time: 1.194581 seconds	 Frequency: 6012000.000000 Hz

Time: 1.244515 seconds	 Frequency: 6011000.000000 Hz

Time: 1.294334 seconds	 Frequency: 6010000.000000 Hz

Time: 1.343890 seconds	 Frequency: 6009333.500000 Hz

Time: 1.393791 seconds	 Frequency: 6010333.500000 Hz

Time: 1.445234 seconds	 Frequency: 6011000.000000 Hz

Time: 1.496608 seconds	 Frequency: 6012000.000000 Hz

Time: 1.547906 seconds	 Frequency: 6012666.500000 Hz

Time: 1.599394 seconds	 Frequency: 6013000.000000 Hz

Time: 1.650434 seconds	 Frequency: 6014000.000000 Hz

Time: 1.700587 seconds	 Frequency: 6014000.000000 Hz

Time: 1.753064 seconds	 Frequency: 6013000.000000 Hz

Time: 1.806254 seconds	 Frequency: 6012333.500000 Hz

Time: 1.857512 seconds	 Frequency: 6013333.500000 Hz

Time: 1.908041 seconds	 Frequency: 6013666.500000 Hz

Time: 1.959692 seconds	 Frequency: 6012666.500000 Hz

Time: 2.010986 seconds	 Frequency: 6012000.000000 Hz

Time: 2.062536 seconds	 Frequency: 6012000.000000 Hz

Time: 2.112330 seconds	 Frequency: 6011666.500000 Hz

Time: 2.162920 seconds	 Frequency: 6012000.000000 Hz

Time: 2.215679 seconds	 Frequency: 6010666.500000 Hz

Time: 2.268756 seconds	 Frequency: 6013666.500000 Hz

Time: 2.320295 seconds	 Frequency: 6013666.500000 Hz

Time: 2.371073 seconds	 Frequency: 6013666.500000 Hz

Time: 2.421083 seconds	 Frequency: 6013666.500000 Hz

Time: 2.471774 seconds	 Frequency: 6013333.500000 Hz

Time: 2.522062 seconds	 Frequency: 6014333.500000 Hz

Time: 2.572713 seconds	 Frequency: 6014000.000000 Hz

Time: 2.625017 seconds	 Frequency: 6013666.500000 Hz

Time: 2.676470 seconds	 Frequency: 6013333.500000 Hz

Time: 2.728208 seconds	 Frequency: 6012666.500000 Hz

Time: 2.779658 seconds	 Frequency: 6012333.500000 Hz

Time: 2.829602 seconds	 Frequency: 6013000.000000 Hz

Time: 2.881583 seconds	 Frequency: 6012666.500000 Hz

Time: 2.933388 seconds	 Frequency: 6011666.500000 Hz

Time: 2.983983 seconds	 Frequency: 6011666.500000 Hz

Time: 3.035952 seconds	 Frequency: 6011333.500000 Hz

Time: 3.087316 seconds	 Frequency: 6012333.500000 Hz

Time: 3.138475 seconds	 Frequency: 6012666.500000 Hz

Time: 3.189550 seconds	 Frequency: 6013333.500000 Hz

Time: 3.240899 seconds	 Frequency: 6013333.500000 Hz

Time: 3.292351 seconds	 Frequency: 6013666.500000 Hz

Time: 3.344785 seconds	 Frequency: 6013333.500000 Hz

Time: 3.394881 seconds	 Frequency: 6014000.000000 Hz

Time: 3.448190 seconds	 Frequency: 6013666.500000 Hz

Time: 3.500576 seconds	 Frequency: 6011333.500000 Hz

Time: 3.550605 seconds	 Frequency: 6012000.000000 Hz

Time: 3.601244 seconds	 Frequency: 6012666.500000 Hz

Time: 3.651385 seconds	 Frequency: 6012333.500000 Hz

Time: 3.701870 seconds	 Frequency: 6011666.500000 Hz

Time: 3.754856 seconds	 Frequency: 6011333.500000 Hz

Time: 3.806478 seconds	 Frequency: 6012333.500000 Hz

Time: 3.856804 seconds	 Frequency: 6012000.000000 Hz

Time: 3.908468 seconds	 Frequency: 6013000.000000 Hz

Time: 3.959929 seconds	 Frequency: 6013333.500000 Hz

Time: 4.010896 seconds	 Frequency: 6013333.500000 Hz

Time: 4.062061 seconds	 Frequency: 6013333.500000 Hz

Time: 4.112705 seconds	 Frequency: 6012666.500000 Hz

Time: 4.164104 seconds	 Frequency: 6012333.500000 Hz

Time: 4.215932 seconds	 Frequency: 6013000.000000 Hz

Time: 4.266767 seconds	 Frequency: 6012333.500000 Hz

Time: 4.316562 seconds	 Frequency: 6012000.000000 Hz

Time: 4.366514 seconds	 Frequency: 6013000.000000 Hz

Time: 4.416383 seconds	 Frequency: 6012333.500000 Hz

Time: 4.470039 seconds	 Frequency: 6011000.000000 Hz

Time: 4.520820 seconds	 Frequency: 6011000.000000 Hz

Time: 4.572038 seconds	 Frequency: 6009666.500000 Hz

Time: 4.624506 seconds	 Frequency: 6007666.500000 Hz

Time: 4.675418 seconds	 Frequency: 6006666.500000 Hz

Time: 4.728178 seconds	 Frequency: 6004666.500000 Hz

Time: 4.779814 seconds	 Frequency: 6001333.500000 Hz

Time: 4.831486 seconds	 Frequency: 5997666.500000 Hz

Time: 4.882470 seconds	 Frequency: 5996333.500000 Hz

Time: 4.934258 seconds	 Frequency: 5996000.000000 Hz

Time: 4.984772 seconds	 Frequency: 5995666.500000 Hz

Time: 5.034955 seconds	 Frequency: 5996000.000000 Hz

Time: 5.086798 seconds	 Frequency: 5998000.000000 Hz

Time: 5.137066 seconds	 Frequency: 5998000.000000 Hz

Time: 5.188338 seconds	 Frequency: 5999666.500000 Hz

Time: 5.241032 seconds	 Frequency: 6001000.000000 Hz

Time: 5.292459 seconds	 Frequency: 6001333.500000 Hz

Time: 5.345051 seconds	 Frequency: 6000333.500000 Hz

Time: 5.395560 seconds	 Frequency: 6000000.000000 Hz

Time: 5.448254 seconds	 Frequency: 5999333.500000 Hz

Time: 5.498697 seconds	 Frequency: 5998333.500000 Hz

Time: 5.550626 seconds	 Frequency: 5996333.500000 Hz

Time: 5.601740 seconds	 Frequency: 5995333.500000 Hz

Time: 5.654720 seconds	 Frequency: 5995000.000000 Hz

Time: 5.705755 seconds	 Frequency: 5995666.500000 Hz

Time: 5.756028 seconds	 Frequency: 5995666.500000 Hz

Time: 5.806159 seconds	 Frequency: 5995666.500000 Hz

Time: 5.856689 seconds	 Frequency: 5996666.500000 Hz

Time: 5.906844 seconds	 Frequency: 5996333.500000 Hz

Time: 5.956937 seconds	 Frequency: 5995333.500000 Hz

Time: 6.007191 seconds	 Frequency: 5995666.500000 Hz

Time: 6.057622 seconds	 Frequency: 5996333.500000 Hz

Time: 6.108585 seconds	 Frequency: 5997333.500000 Hz

Time: 6.159243 seconds	 Frequency: 5997666.500000 Hz

Time: 6.209434 seconds	 Frequency: 5997333.500000 Hz

Time: 6.260083 seconds	 Frequency: 5998000.000000 Hz

Time: 6.311165 seconds	 Frequency: 5997666.500000 Hz

Time: 6.363252 seconds	 Frequency: 5997666.500000 Hz

Time: 6.414091 seconds	 Frequency: 5996666.500000 Hz

Time: 6.464848 seconds	 Frequency: 5996666.500000 Hz

Time: 6.515979 seconds	 Frequency: 5996666.500000 Hz

Time: 6.567700 seconds	 Frequency: 5996666.500000 Hz

Time: 6.617520 seconds	 Frequency: 5997000.000000 Hz

Time: 6.667756 seconds	 Frequency: 5997000.000000 Hz

Time: 6.718293 seconds	 Frequency: 5996000.000000 Hz

Time: 6.768389 seconds	 Frequency: 5996000.000000 Hz

Time: 6.818945 seconds	 Frequency: 5997000.000000 Hz

Time: 6.869511 seconds	 Frequency: 5996666.500000 Hz

Time: 6.920420 seconds	 Frequency: 5996666.500000 Hz

Time: 6.971357 seconds	 Frequency: 5996666.500000 Hz

Time: 7.022105 seconds	 Frequency: 5996666.500000 Hz

Time: 7.072647 seconds	 Frequency: 5996333.500000 Hz

Time: 7.124744 seconds	 Frequency: 5996333.500000 Hz

Time: 7.175262 seconds	 Frequency: 5996666.500000 Hz

Time: 7.225921 seconds	 Frequency: 5996666.500000 Hz

Time: 7.277148 seconds	 Frequency: 5996000.000000 Hz

Time: 7.328028 seconds	 Frequency: 5995333.500000 Hz

Time: 7.378528 seconds	 Frequency: 5995333.500000 Hz

Time: 7.428275 seconds	 Frequency: 5996000.000000 Hz

Time: 7.478247 seconds	 Frequency: 5996333.500000 Hz

Time: 7.528361 seconds	 Frequency: 5995666.500000 Hz

Time: 7.579563 seconds	 Frequency: 5995666.500000 Hz

Time: 7.629555 seconds	 Frequency: 5996000.000000 Hz

Time: 7.679929 seconds	 Frequency: 5995333.500000 Hz

Time: 7.730001 seconds	 Frequency: 5996000.000000 Hz

Time: 7.781161 seconds	 Frequency: 5996000.000000 Hz

Time: 7.832209 seconds	 Frequency: 5996333.500000 Hz

Time: 7.883713 seconds	 Frequency: 5995333.500000 Hz

Time: 7.935777 seconds	 Frequency: 5996000.000000 Hz

Time: 7.986828 seconds	 Frequency: 5996666.500000 Hz

Time: 8.038172 seconds	 Frequency: 5995666.500000 Hz

Time: 8.089030 seconds	 Frequency: 5996666.500000 Hz

Time: 8.141976 seconds	 Frequency: 5996333.500000 Hz

Time: 8.192905 seconds	 Frequency: 5995333.500000 Hz

Time: 8.241948 seconds	 Frequency: 5996333.500000 Hz

Time: 8.292240 seconds	 Frequency: 5996000.000000 Hz

Time: 8.342078 seconds	 Frequency: 5995666.500000 Hz

Time: 8.392787 seconds	 Frequency: 5996000.000000 Hz

Time: 8.445452 seconds	 Frequency: 5996000.000000 Hz

Time: 8.497008 seconds	 Frequency: 5995666.500000 Hz

Time: 8.547763 seconds	 Frequency: 5995666.500000 Hz

Time: 8.598746 seconds	 Frequency: 5995666.500000 Hz

Time: 8.650582 seconds	 Frequency: 5995666.500000 Hz

Time: 8.701841 seconds	 Frequency: 5995333.500000 Hz

Time: 8.755038 seconds	 Frequency: 5995666.500000 Hz

Time: 8.806604 seconds	 Frequency: 5995666.500000 Hz

Time: 8.857604 seconds	 Frequency: 5995333.500000 Hz

Time: 8.909235 seconds	 Frequency: 5994666.500000 Hz

Time: 8.961700 seconds	 Frequency: 5995333.500000 Hz

Time: 9.012679 seconds	 Frequency: 5996000.000000 Hz

Time: 9.063740 seconds	 Frequency: 5996333.500000 Hz

Time: 9.115460 seconds	 Frequency: 5996000.000000 Hz

Time: 9.168563 seconds	 Frequency: 5997000.000000 Hz

Time: 9.220975 seconds	 Frequency: 5997333.500000 Hz

Time: 9.271726 seconds	 Frequency: 5997333.500000 Hz

Time: 9.321791 seconds	 Frequency: 5997000.000000 Hz

Time: 9.372070 seconds	 Frequency: 5997333.500000 Hz

Time: 9.423537 seconds	 Frequency: 5997000.000000 Hz

Time: 9.476287 seconds	 Frequency: 5996000.000000 Hz

Time: 9.527560 seconds	 Frequency: 5996000.000000 Hz

Time: 9.578807 seconds	 Frequency: 5996000.000000 Hz

Time: 9.630452 seconds	 Frequency: 5996000.000000 Hz

Time: 9.682372 seconds	 Frequency: 5996333.500000 Hz

Time: 9.733722 seconds	 Frequency: 5997000.000000 Hz

Time: 9.785492 seconds	 Frequency: 5996666.500000 Hz

Time: 9.836668 seconds	 Frequency: 5996666.500000 Hz

Time: 9.887619 seconds	 Frequency: 5997333.500000 Hz

Time: 9.937988 seconds	 Frequency: 5996000.000000 Hz

Time: 9.990212 seconds	 Frequency: 5996666.500000 Hz

Time: 10.042130 seconds	 Frequency: 5996333.500000 Hz

Exercise 2 TI-RTOS

Exercise 2.1 Multitask programming to blink LEDs

1.)

Red LED delay value: 250 msec

Green LED delay value: 1000 msec

2.)

In the following code, I set up the red LED as an output pin. Then I have a forever loop that toggles the output on the pin in order to blink the LED, and I use the Task_sleep() function to delay for 250 msec between each toggle of the LED.

// blink red LED at 2 Hz

Void task1(UArg arg0, UArg arg1)

{

printf("Task1\n");

//T = 1/F

//T = 1/2

//T = 0.5 secs / 2 for on and off

//T = 0.25 secs * 1000 = 250 msec

// set up red LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

while(1)

{

//toggle output on red LED pin

GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

//delay for time calculated earlier

Task_sleep(2500);

}

}

Exercise 2.2 Single task function

1.)

In the following code I define a structure that contains the information for each task including the port number, pin number, the specified delay and the LED color. I then create an array of the defined structure type in order to hold information for each of the tasks. In order to pass this array information to the function, when constructing the task threads, I changed the arg0 to the respective index, 0 for the function to access the red LED information, and 1 to access the green LED information. In the common task function I then access the index of the global config array using the argument passed to the function, in order to get the correct information to control the LED for each task.

// create structure for LED information

typedef struct

{

uint_fast8_t port;

uint_fast16_t pin;

uint32_t delay_ms;

char *LED;

} LED_Info;

// create array of the structure defined above that contains the info for each LED

LED_Info config[2] = {{GPIO_PORT_P2, GPIO_PIN0, 250, "Red LED"}, {GPIO_PORT_P2, GPIO_PIN1, 1000, "Green LED"}};

/* Construct task threads */

Task_Params_init(&taskParams);

taskParams.stackSize = TASKSTACKSIZE;

taskParams.stack = &task1Stack;

taskParams.arg0 = 0;

Task_construct(&task1Struct, (Task_FuncPtr)common_task, &taskParams, NULL);

taskParams.stack = &task2Stack;

taskParams.arg0 = 1;

Task_construct(&task2Struct, (Task_FuncPtr)common_task, &taskParams, NULL);

// common task for LED control

Void common_task(UArg arg0, UArg arg1)

{

// arg0 contains the index to the LED info

LED_Info* info = &config[arg0];

// print LED info for task

printf("Starting task for %s\n", info->LED);

printf("Port: %d, Pin: %d, Delay: %d ms\n", info->port, info->pin, info->delay_ms);

// set up LED pin

MAP_GPIO_setAsOutputPin(info->port, info->pin);

// forever loop to blink LED

while(1)

{

GPIO_toggleOutputOnPin(info->port, info->pin);

Task_sleep(info->delay_ms);

}

}

2.)

[CORTEX_M4_0] Starting task for Red LED

Port: 2, Pin: 1, Delay: 250 ms

Starting task for Green LED

Port: 2, Pin: 2, Delay: 1000 ms

Appendix

Exercise 1

Exercise 1.1 Program

// Program to digitize the MCU's internal temperature sensor's output voltage

// DriverLib Includes

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

// Standard Includes

#include <stdint.h>

#include <stdio.h>

#include <string.h>

volatile bool adc_done;

int main(void)

{

// Halt WDT

WDT_A_holdTimer();

printf("MCLK: %u\n", MAP_CS_getMCLK());

// Set reference voltage to 1.2 V and enable temperature sensor

REF_A_enableReferenceVoltage();

REF_A_enableTempSensor();

REF_A_setReferenceVoltage(REF_A_VREF1_2V);

// Initializing ADC (MCLK/1/1) with temperature sensor routed

ADC14_enableModule();

ADC14_initModule(ADC_CLOCKSOURCE_MCLK, ADC_PREDIVIDER_1, ADC_DIVIDER_1, ADC_TEMPSENSEMAP);

// Set resolution

ADC14_setResolution(ADC_14BIT);

// Configure ADC Memory for temperature sensor data

ADC14_configureSingleSampleMode(ADC_MEM0, false);

ADC14_configureConversionMemory(ADC_MEM0, ADC_VREFPOS_INTBUF_VREFNEG_VSS, ADC_INPUT_A22, false);

// Enable sample timer in manual iteration mode and interrupts

ADC14_enableSampleTimer(ADC_MANUAL_ITERATION);

// Enable conversion

ADC14_enableConversion();

// Enabling Interrupts

ADC14_enableInterrupt(ADC_INT0);

Interrupt_enableInterrupt(INT_ADC14);

Interrupt_enableMaster();

while(1)

{

// Trigger conversion with software

adc_done = false;

ADC14_toggleConversionTrigger();

while (!adc_done)

{

}

uint32_t adc_value = ADC14_getResult(ADC_MEM0);

printf("%u", adc_value);

float voltage_in_mv = (1.2 * (float) adc_value * 1000) / 16384;

printf("\tVoltage: %.2f mV\n", voltage_in_mv);

__delay_cycles(1500000);

}

}

// This interrupt happens every time a conversion has completed

void ADC14_IRQHandler(void)

{

uint64_t status;

status = ADC14_getEnabledInterruptStatus();

ADC14_clearInterruptFlag(status);

if(status & ADC_INT0)

{

adc_done = true;

}

}

Exercise 1.2 Program

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define SAMPLING_FREQUENCY 100 // Sampling frequency in Hz

/* Timer_A Continuous Mode Configuration Parameter */

const Timer_A_UpModeConfig upModeConfig =

{

TIMER_A_CLOCKSOURCE_SMCLK,            // SMCLK Clock Source

TIMER_A_CLOCKSOURCE_DIVIDER_1,       // SMCLK/1 = 3 MHz

(3000000 / SAMPLING_FREQUENCY),

TIMER_A_TAIE_INTERRUPT_DISABLE,      // Disable Timer ISR

TIMER_A_CCIE_CCR0_INTERRUPT_DISABLE, // Disable CCR0

TIMER_A_DO_CLEAR                     // Clear Counter

};

/* Timer_A Compare Configuration Parameter */

const Timer_A_CompareModeConfig compareConfig =

{

TIMER_A_CAPTURECOMPARE_REGISTER_1,          // Use CCR1

TIMER_A_CAPTURECOMPARE_INTERRUPT_DISABLE,   // Disable CCR interrupt

TIMER_A_OUTPUTMODE_SET_RESET,               // Toggle output but

(3000000 / SAMPLING_FREQUENCY)    // 16000 Period

};

static volatile uint32_t adc_value;

static volatile bool adc_done;

int main(void)

{

/* Halting WDT  */

MAP_WDT_A_holdTimer();

// Set reference voltage to 1.2 V and enable temperature sensor

REF_A_enableReferenceVoltage();

REF_A_enableTempSensor();

REF_A_setReferenceVoltage(REF_A_VREF1_2V);

// Initializing ADC (MCLK/1/1) with temperature sensor routed

ADC14_enableModule();

ADC14_initModule(ADC_CLOCKSOURCE_MCLK, ADC_PREDIVIDER_1, ADC_DIVIDER_1, ADC_TEMPSENSEMAP);

// Set resolution

ADC14_setResolution(ADC_14BIT);

// Configure the sample/hold time

ADC14_setSampleHoldTime(ADC_PULSE_WIDTH_192, ADC_PULSE_WIDTH_192);

/* Configuring GPIOs (5.5 A0) */

MAP_GPIO_setAsPeripheralModuleFunctionInputPin(GPIO_PORT_P5, GPIO_PIN5,

GPIO_TERTIARY_MODULE_FUNCTION);

/* Configuring ADC Memory */

MAP_ADC14_configureSingleSampleMode(ADC_MEM0, true);

MAP_ADC14_configureConversionMemory(ADC_MEM0, ADC_VREFPOS_INTBUF_VREFNEG_VSS, ADC_INPUT_A22, false);

/* Configuring Timer_A in continuous mode and sourced from ACLK */

MAP_Timer_A_configureUpMode(TIMER_A0_BASE, &upModeConfig);

/* Configuring Timer_A0 in CCR1 to trigger at 16000 (0.5s) */

MAP_Timer_A_initCompare(TIMER_A0_BASE, &compareConfig);

/* Configuring the sample trigger to be sourced from Timer_A0  and setting it

* to automatic iteration after it is triggered*/

MAP_ADC14_setSampleHoldTrigger(ADC_TRIGGER_SOURCE1, false);

/* Enabling the interrupt when a conversion on channel 1 is complete and

* enabling conversions */

MAP_ADC14_enableInterrupt(ADC_INT0);

MAP_ADC14_enableConversion();

/* Enabling Interrupts */

MAP_Interrupt_enableInterrupt(INT_ADC14);

MAP_Interrupt_enableMaster();

// Variables for frequency measurement

uint32_t sample_count = 0;

uint32_t t0, t1;

bool measuring = true;

float measured_freq;

// Start Timer32 for frequency measurement

Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

Timer32_startTimer(TIMER32_0_BASE, 0);

/* Starting the Timer */

MAP_Timer_A_startCounter(TIMER_A0_BASE, TIMER_A_UP_MODE);

// Get start time

t0 = Timer32_getValue(TIMER32_0_BASE);

// set conversion bool variable to false

adc_done = false;

// Measure frequency

while(measuring)

{

// if conversion is done

if(adc_done)

{

// increment sample count

sample_count++;

// set conversion bool variable to false

adc_done = false;

// once sample count reaches 10

if(sample_count >= 10)

{

// get end timer value

t1 = Timer32_getValue(TIMER32_0_BASE);

// calculate difference in timer values

uint32_t time_diff = t0 - t1;

// calculate elapsed time in seconds

float elapsed_time = (float)time_diff / 3000000;

// calculate the frequency

measured_freq = (float)sample_count / elapsed_time;

printf("Measured sampling frequency: %.2f Hz\n", measured_freq);

// set measuring to false to signify end of measurement

measuring = false;

}

}

}

while (1)

{

printf("%u", adc_value);

float voltage_in_mv = (1.2 * adc_value * 1000) / 16384;

printf("\tVoltage: %.2f mV\n", voltage_in_mv);

__delay_cycles(1500000);

}

}

/* This interrupt is fired whenever a conversion is completed and placed in

* ADC_MEM0 */

void ADC14_IRQHandler(void)

{

uint64_t status;

status = MAP_ADC14_getEnabledInterruptStatus();

MAP_ADC14_clearInterruptFlag(status);

if (status & ADC_INT0)

{

adc_value = ADC14_getResult(ADC_MEM0);

adc_done = true;

}

}

Exercise 1.3 Program

Lab 7 Program

// Program to digitize the MCU's internal temperature sensor's output voltage

// DriverLib Includes

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

// Standard Includes

#include <stdint.h>

#include <stdio.h>

#include <string.h>

volatile bool adc_done;

int main(void)

{

// Halt WDT

WDT_A_holdTimer();

printf("MCLK: %u\n", MAP_CS_getMCLK());

// Set reference voltage to 1.2 V and enable temperature sensor

REF_A_enableReferenceVoltage();

REF_A_enableTempSensor();

REF_A_setReferenceVoltage(REF_A_VREF1_2V);

// Initializing ADC (MCLK/1/1) with temperature sensor routed

ADC14_enableModule();

ADC14_initModule(ADC_CLOCKSOURCE_MCLK, ADC_PREDIVIDER_1, ADC_DIVIDER_1, ADC_TEMPSENSEMAP);

// Set resolution

ADC14_setResolution(ADC_14BIT);

// Configure ADC Memory for temperature sensor data

ADC14_configureSingleSampleMode(ADC_MEM0, false);

ADC14_configureConversionMemory(ADC_MEM0, ADC_VREFPOS_INTBUF_VREFNEG_VSS, ADC_INPUT_A22, false);

// Enable sample timer in manual iteration mode and interrupts

ADC14_enableSampleTimer(ADC_MANUAL_ITERATION);

// Enable conversion

ADC14_enableConversion();

// Enabling Interrupts

ADC14_enableInterrupt(ADC_INT0);

Interrupt_enableInterrupt(INT_ADC14);

Interrupt_enableMaster();

uint_fast16_t calib_30 = SysCtl_getTempCalibrationConstant(SYSCTL_1_2V_REF, SYSCTL_30_DEGREES_C);

uint_fast16_t calib_85 = SysCtl_getTempCalibrationConstant(SYSCTL_1_2V_REF, SYSCTL_85_DEGREES_C);

printf("Calibration values: 30 C: %u, 85 C: %u\n", calib_30, calib_85);

while(1)

{

// Trigger conversion with software

adc_done = false;

ADC14_toggleConversionTrigger();

while (!adc_done)

{

}

uint32_t adc_value = ADC14_getResult(ADC_MEM0);

printf("%u", adc_value);

float voltage_in_mv = (1.2 * (float) adc_value * 1000) / 16384;

printf("\tVoltage: %.2f mV", voltage_in_mv);

// Calculate temperature using calibration points

float slope = (float)(calib_85 - calib_30) / 55;

float intercept = calib_85 - slope * 85;

float temp_c = (adc_value - intercept) / slope;

float temp_f = (temp_c * 9 / 5) + 32.0;

printf("\tTemperature: %.2f F\n", temp_f);

__delay_cycles(1500000);

}

}

// This interrupt happens every time a conversion has completed

void ADC14_IRQHandler(void)

{

uint64_t status;

status = ADC14_getEnabledInterruptStatus();

ADC14_clearInterruptFlag(status);

if(status & ADC_INT0)

{

adc_done = true;

}

}

Lab 5 Program for frequency measurement

/* DriverLib Includes */

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

/* Standard Includes */

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

#define SAMPLES 100

//define min and max timer values

#define MAX_TIMER_VAL 0xFFFFFFFF

#define MIN_TIMER_VAL 0xFFFF0000

void delay_ms(uint32_t count);

int main(void)

{

/* Stop Watchdog  */

MAP_WDT_A_holdTimer();

//initialize 32 bit timer

MAP_Timer32_initModule(TIMER32_0_BASE, TIMER32_PRESCALER_1, TIMER32_32BIT, TIMER32_FREE_RUN_MODE);

MAP_Timer32_startTimer(TIMER32_0_BASE, 0);

CAPTIO0CTL = 0; // Clear control register

CAPTIO0CTL |= 0x0100; // Enable CAPTIO

CAPTIO0CTL |= 4 << 4; // Select Port 4. Place port number in Bits 7-4

CAPTIO0CTL |= 1 << 1; // Select Pin 1. Place pin number in Bits 3-1

Timer_A_ContinuousModeConfig timer_continuous_obj;

timer_continuous_obj.clockSource = TIMER_A_CLOCKSOURCE_INVERTED_EXTERNAL_TXCLK;

timer_continuous_obj.clockSourceDivider = TIMER_A_CLOCKSOURCE_DIVIDER_1;

timer_continuous_obj.timerInterruptEnable_TAIE =

TIMER_A_TAIE_INTERRUPT_DISABLE;

timer_continuous_obj.timerClear = TIMER_A_DO_CLEAR;

MAP_Timer_A_configureContinuousMode(TIMER_A2_BASE, &timer_continuous_obj);

MAP_Timer_A_startCounter(TIMER_A2_BASE, TIMER_A_CONTINUOUS_MODE);

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

uint32_t counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

float freq = (float)counter / (3.0/1000.0);

printf("Frequency: %f\n", freq);

//get clock speed

uint32_t clk = MAP_CS_getMCLK();

//calculate how many cycles is in 10 seconds to use in while loop

uint32_t ten_secs = clk * 10;

//get timer value at start of measurement loop

uint32_t t0 = MAP_Timer32_getValue(TIMER32_0_BASE);

while(1)

{

//clear the counter

MAP_Timer_A_clearTimer(TIMER_A2_BASE);

//delay for 3 ms

delay_ms(3);

//get counter value

counter = MAP_Timer_A_getCounterValue(TIMER_A2_BASE);

//calculate frequency

freq = (float)counter / (3.0/1000.0);

//get timer value at end of measurement

uint32_t t1 = MAP_Timer32_getValue(TIMER32_0_BASE);

float time_in_s = (t0 - t1) / (float)clk;

printf("Time: %f seconds\t Frequency: %f Hz\n", time_in_s, freq);

fflush(stdout);

//check if time elapsed is greater than 10 seconds

if((t0 - t1) >= ten_secs)

{

break;

}

}

while(1);

}

//function to delay for specified number of ms

void delay_ms(uint32_t count)

{

//get prev value of timer

uint32_t prev = MAP_Timer32_getValue(TIMER32_0_BASE);

//calculate number of cycles required to delay specified # of ms

uint64_t num_delay_cycles = ((uint64_t)MAP_CS_getMCLK() * count) / 1000;

uint64_t counter = 0;

//loop until counter reaches the number of delay cycles

while(num_delay_cycles > counter)

{

//get current timer value

uint32_t current = MAP_Timer32_getValue(TIMER32_0_BASE);

//check if timer has wrapped around

if(current > prev)

{

//if it has add to the counter (difference of min timer val and prev plus the difference of current timer val and max timer val)

counter += ((prev - MIN_TIMER_VAL) + (MAX_TIMER_VAL - current));

}

else

{

//if it hasn't wrapped around just add to the counter the difference of prev and current values

counter += (prev - current);

}

//set prev timer val equal to current timer value

prev = current;

}

}

Exercise 2

Exercise 2.1 Program

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#include <string.h>

/* XDC module Headers */

#include <xdc/std.h>

#include <xdc/runtime/System.h>

/* BIOS module Headers */

#include <ti/sysbios/BIOS.h>

#include <ti/sysbios/knl/Clock.h>

#include <ti/sysbios/knl/Task.h>

#include <ti/sysbios/knl/Semaphore.h>

#include <ti/drivers/Board.h>

#define __MSP432P4XX__

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#define TASKSTACKSIZE   2048

Void clockHandler1(UArg arg);

Void task1(UArg arg0, UArg arg1);

Void task2(UArg arg0, UArg arg1);

Task_Struct task1Struct, task2Struct;

Char task1Stack[TASKSTACKSIZE], task2Stack[TASKSTACKSIZE];

int main()

{

/* Construct BIOS objects */

Task_Params taskParams;

/* Call driver init functions */

Board_init();

/* Construct task threads */

Task_Params_init(&taskParams);

taskParams.stackSize = TASKSTACKSIZE;

taskParams.stack = &task1Stack;

Task_construct(&task1Struct, (Task_FuncPtr)task1, &taskParams, NULL);

taskParams.stack = &task2Stack;

Task_construct(&task2Struct, (Task_FuncPtr)task2, &taskParams, NULL);

// Create clock that calls Task_yield() to preempt tasks

Clock_Params clockParams;

Clock_Handle myClk0;

Clock_Params_init(&clockParams);

clockParams.period = 10;         // 1 Clock tick is 1 ms by default

clockParams.startFlag = TRUE;    // Start immediately

myClk0 = Clock_create((Clock_FuncPtr)clockHandler1, 1, &clockParams, NULL);   // 2nd argument must be greater than 0

if (myClk0 == NULL)

System_abort("Clock0 create failed");

BIOS_start();    /* Does not return */

return(0);

}

// blink red LED at 2 Hz

Void task1(UArg arg0, UArg arg1)

{

printf("Task1\n");

//T = 1/F

//T = 1/2

//T = 0.5 secs / 2 for on and off

//T = 0.25 secs * 1000 = 250 msec

// set up red LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

while(1)

{

//toggle output on red LED pin

GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

//delay for time calculated earlier

Task_sleep(250);

}

}

// blink green LED at 0.5 Hz

Void task2(UArg arg0, UArg arg1)

{

printf("Task2\n");

//T = 1/F

//T = 1/0.5

//T = 2 secs / 2 for on and off

//T = 1 secs * 1000 = 1000 msec

// set up green LED

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN1);

while(1)

{

//toggle output on green LED pin

GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN1);

//delay for time calculated earlier

Task_sleep(1000);

}

}

Void clockHandler1(UArg arg)

{

Task_yield();                   // To force a task switch if needed

}

Exercise 2.2 Program

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#include <string.h>

/* XDC module Headers */

#include <xdc/std.h>

#include <xdc/runtime/System.h>

/* BIOS module Headers */

#include <ti/sysbios/BIOS.h>

#include <ti/sysbios/knl/Clock.h>

#include <ti/sysbios/knl/Task.h>

#include <ti/sysbios/knl/Semaphore.h>

#include <ti/drivers/Board.h>

#define __MSP432P4XX__

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#define TASKSTACKSIZE   2048

Void clockHandler1(UArg arg);

Void common_task(UArg arg0, UArg arg1);

Task_Struct task1Struct, task2Struct;

Char task1Stack[TASKSTACKSIZE], task2Stack[TASKSTACKSIZE];

// create structure for LED information

typedef struct

{

uint_fast8_t port;

uint_fast16_t pin;

uint32_t delay_ms;

char *LED;

} LED_Info;

// create array of the structure defined above that contains the info for each LED

LED_Info config[2] = {{GPIO_PORT_P2, GPIO_PIN0, 250, "Red LED"}, {GPIO_PORT_P2, GPIO_PIN1, 1000, "Green LED"}};

int main()

{

/* Construct BIOS objects */

Task_Params taskParams;

/* Call driver init functions */

Board_init();

/* Construct task threads */

Task_Params_init(&taskParams);

taskParams.stackSize = TASKSTACKSIZE;

taskParams.stack = &task1Stack;

taskParams.arg0 = 0;

Task_construct(&task1Struct, (Task_FuncPtr)common_task, &taskParams, NULL);

taskParams.stack = &task2Stack;

taskParams.arg0 = 1;

Task_construct(&task2Struct, (Task_FuncPtr)common_task, &taskParams, NULL);

// Create clock that calls Task_yield() to preempt tasks

Clock_Params clockParams;

Clock_Handle myClk0;

Clock_Params_init(&clockParams);

clockParams.period = 10;         // 1 Clock tick is 1 ms by default

clockParams.startFlag = TRUE;    // Start immediately

myClk0 = Clock_create((Clock_FuncPtr)clockHandler1, 1, &clockParams, NULL);   // 2nd argument must be greater than 0

if (myClk0 == NULL)

System_abort("Clock0 create failed");

BIOS_start();    /* Does not return */

return(0);

}

// common task for LED control

Void common_task(UArg arg0, UArg arg1)

{

// arg0 contains the index to the LED info

LED_Info* info = &config[arg0];

// print LED info for task

printf("Starting task for %s\n", info->LED);

printf("Port: %d, Pin: %d, Delay: %d ms\n", info->port, info->pin, info->delay_ms);

// set up LED pin

MAP_GPIO_setAsOutputPin(info->port, info->pin);

// forever loop to blink LED

while(1)

{

GPIO_toggleOutputOnPin(info->port, info->pin);

Task_sleep(info->delay_ms);

}

}

Void clockHandler1(UArg arg)

{

Task_yield();                   // To force a task switch if needed

}