Ryan Smith

015792504

CMPE 146 Sec 01

9 December 2024

Lab 8

Exercise 1 Semaphore

1.)

Void task1(UArg arg0, UArg arg1)

{

while(1)

{

// increase semaphore's count value

Semaphore_post(semaphoreHandle);

// delay blinking frequency to blink at 0.5 Hz

Task_sleep(1000);

}

}

Void task2(UArg arg0, UArg arg1)

{

// set red LED as output pin

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

while(1)

{

// try to get semaphore

Semaphore_pend(semaphoreHandle, BIOS_WAIT_FOREVER);

// toggle red LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

}

}

2.)

In order to control the blinking, task1 increments the semaphore’s count. This allows task2 to proceed as it has called Semaphore_pend to wait for the semaphore once the count is not 0. It then toggles the output on the pin. Task1 has a delay that runs before it can call Semaphore_post again in order to blink the LED at a rate of 0.5 Hz.

3.)

In order to make that not happen, we can set a maximum semaphore count value of 1 which would essentially be a binary semaphore.

Exercise 2 Multiple Tasks to Compute

1.)

In order to synchronize the tasks, the counting semaphore is initialized to 0 in main. Then, in each task, after it calculates the partial factorial, it calls Semaphore_post to increment the count. In the master task, it calls Semaphore_pend for NTASKS in order to wait for all tasks to complete, which means it waits for each task to increment the semaphore count. Once the count is incremented and all Semaphore_pend calls have been made it does not have to wait anymore and can calculate the final factorial result.

2.)

[CORTEX_M4_0] Input parameters for task 0:	 start: 1 	 end: 55

Input parameters for task 1:	 start: 56 	 end: 110

Input parameters for task 2:	 start: 111 	 end: 165

Task: 0 	 Completed 10 multiplications

Task: 1 	 Completed 10 multiplications

Task: 2 	 Completed 10 multiplications

Task: 0 	 Completed 10 multiplications

Task: 1 	 Completed 10 multiplications

Task: 2 	 Completed 10 multiplications

Task: 0 	 Completed 10 multiplications

Task: 1 	 Completed 10 multiplications

Task: 2 	 Completed 10 multiplications

Task: 0 	 Completed 10 multiplications

Task: 1 	 Completed 10 multiplications

Task: 2 	 Completed 10 multiplications

Task: 0 	 Completed 10 multiplications

Task: 1 	 Completed 10 multiplications

Task: 2 	 Completed 10 multiplications

Task: 0 complete

Task: 1 complete

Task: 2 complete

Final factorial: 5.433419e+289

Appendix

Exercise 1 Program

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#include <string.h>

/* XDC module Headers */

#include <xdc/std.h>

#include <xdc/runtime/System.h>

#include <xdc/runtime/Timestamp.h>

#include <xdc/runtime/Types.h>

/* BIOS module Headers */

#include <ti/sysbios/BIOS.h>

#include <ti/sysbios/knl/Clock.h>

#include <ti/sysbios/knl/Task.h>

#include <ti/sysbios/knl/Semaphore.h>

#include <ti/drivers/Board.h>

#define __MSP432P4XX__

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#define TASKSTACKSIZE   2048

Void clockHandler1(UArg arg);

Void task1(UArg arg0, UArg arg1);

Void task2(UArg arg0, UArg arg1);

Task_Struct task1Struct, task2Struct;

Char task1Stack[TASKSTACKSIZE], task2Stack[TASKSTACKSIZE];

Semaphore_Struct semaStruct;

Semaphore_Handle semaphoreHandle = NULL;

int main()

{

// Init drivers

Board_init();

srand(time(NULL));      // Set seed for random number generator

// Construct tasks

Task_Params taskParams;

Task_Params_init(&taskParams);

taskParams.stackSize = TASKSTACKSIZE;

// Task 1

taskParams.stack = &task1Stack;

Task_construct(&task1Struct, (Task_FuncPtr)task1, &taskParams, NULL);

// Task 2

taskParams.stack = &task2Stack;

Task_construct(&task2Struct, (Task_FuncPtr)task2, &taskParams, NULL);

// Construct a semaphore object

Semaphore_Params semaParams;

Semaphore_Params_init(&semaParams);                 // Initialize structure with default parameters

Semaphore_construct(&semaStruct, 0, &semaParams);   // Create an instance of semaphore object

semaphoreHandle = Semaphore_handle(&semaStruct);

// Create clock that calls Task_yield() to preempt tasks

Clock_Params clockParams;

Clock_Handle myClk0;

Clock_Params_init(&clockParams);

clockParams.period = 10;         // 1 Clock tick is 1 ms by default

clockParams.startFlag = TRUE;    // Start immediately

myClk0 = Clock_create((Clock_FuncPtr)clockHandler1, 1, &clockParams, NULL);   // 2nd argument must be greater than 0

if (myClk0 == NULL)

System_abort("Clock0 create failed");

BIOS_start();    // Jump to the OS and won't return

return(0);

}

Void task1(UArg arg0, UArg arg1)

{

while(1)

{

// increase semaphore's count value

Semaphore_post(semaphoreHandle);

// delay blinking frequency to blink at 0.5 Hz

Task_sleep(1000);

}

}

Void task2(UArg arg0, UArg arg1)

{

// set red LED as output pin

MAP_GPIO_setAsOutputPin(GPIO_PORT_P2, GPIO_PIN0);

while(1)

{

// try to get semaphore

Semaphore_pend(semaphoreHandle, BIOS_WAIT_FOREVER);

// toggle red LED

MAP_GPIO_toggleOutputOnPin(GPIO_PORT_P2, GPIO_PIN0);

}

}

Void clockHandler1(UArg arg)

{

Task_yield();                   // To force a task switch if needed

}

Exercise 2 Program

#include <stdio.h>

#include <stdlib.h>

#include <time.h>

#include <string.h>

/* XDC module Headers */

#include <xdc/std.h>

#include <xdc/runtime/System.h>

#include <xdc/runtime/Timestamp.h>

#include <xdc/runtime/Types.h>

/* BIOS module Headers */

#include <ti/sysbios/BIOS.h>

#include <ti/sysbios/knl/Clock.h>

#include <ti/sysbios/knl/Task.h>

#include <ti/sysbios/knl/Semaphore.h>

#include <ti/drivers/Board.h>

#define __MSP432P4XX__

#include <ti/devices/msp432p4xx/driverlib/driverlib.h>

#define TASKSTACKSIZE   2048

Void clockHandler1(UArg arg);

Void task1(UArg arg0, UArg arg1);

Void task2(UArg arg0, UArg arg1);

Task_Struct task1Struct, task2Struct;

Char task1Stack[TASKSTACKSIZE], task2Stack[TASKSTACKSIZE];

Semaphore_Struct semaStruct;

Semaphore_Handle semaphoreHandle = NULL;

#define FACT_NUM 165

#define NTASKS 3

Task_Struct taskStructs[NTASKS];

Char taskStacks[NTASKS][TASKSTACKSIZE];

struct app_task_params

{

int index;

double partial_result;

int start;

int end;

};

struct app_task_params task_params_list[NTASKS];

Void factorial_task(UArg arg0, UArg arg1);

int main()

{

// Init drivers

Board_init();

srand(time(NULL));      // Set seed for random number generator

// Construct tasks

Task_Params taskParams;

Task_Params_init(&taskParams);

taskParams.stackSize = TASKSTACKSIZE;

// variables to store how many numbers to delegate for each task

uint32_t num_per_task = FACT_NUM/NTASKS;

uint32_t remaining_nums = FACT_NUM % NTASKS;

uint32_t current_num = 1;

uint32_t i;

// loop to create tasks

for(i = 0; i < NTASKS; i++)

{

// set params for each task so each task knows which numbers to calculate factorial for

task_params_list[i].index = i;

task_params_list[i].partial_result = 1.0;

task_params_list[i].start = current_num;

task_params_list[i].end = current_num + num_per_task - 1;

// update current num for next task

current_num = task_params_list[i].end + 1;

// if last task add remaining nums to end

if(i == NTASKS - 1)

{

task_params_list[i].end += remaining_nums;

}

taskParams.stack = &taskStacks[i];

taskParams.arg0 = (UArg)&task_params_list[i];

Task_construct(&taskStructs[i], (Task_FuncPtr)factorial_task, &taskParams, NULL);

}

// Construct a semaphore object

Semaphore_Params semaParams;

Semaphore_Params_init(&semaParams);                 // Initialize structure with default parameters

Semaphore_construct(&semaStruct, 0, &semaParams);   // Create an instance of semaphore object

semaphoreHandle = Semaphore_handle(&semaStruct);

// Create clock that calls Task_yield() to preempt tasks

Clock_Params clockParams;

Clock_Handle myClk0;

Clock_Params_init(&clockParams);

clockParams.period = 10;         // 1 Clock tick is 1 ms by default

clockParams.startFlag = TRUE;    // Start immediately

myClk0 = Clock_create((Clock_FuncPtr)clockHandler1, 1, &clockParams, NULL);   // 2nd argument must be greater than 0

if (myClk0 == NULL)

System_abort("Clock0 create failed");

BIOS_start();    // Jump to the OS and won't return

return(0);

}

Void factorial_task(UArg arg0, UArg arg1)

{

struct app_task_params *params = (struct app_task_params *)arg0;

printf("Input parameters for task %d:", params->index);

printf("\t start: %d \t end: %d\n", params->start, params->end);

// compute partial factorial for task

uint32_t i;

uint32_t progress_count = 0;

for(i = params->start; i < params->end; i++)

{

params->partial_result *= i;

progress_count++;

// for every 10 multiplications print a progress message

if(progress_count == 10)

{

printf("Task: %d \t Completed 10 multiplications\n", params->index);

progress_count = 0;

}

Task_sleep(1);

}

// print message when task is complete

printf("Task: %d complete\n", params->index);

// increment semaphore count to signal end of task

Semaphore_post(semaphoreHandle);

// check if master task

if(params->index == 0)

{

// call semaphore pend for NTASKS in order to wait for all tasks to complete

for(i = 0; i < NTASKS; i++)

{

Semaphore_pend(semaphoreHandle, BIOS_WAIT_FOREVER);

}

// compute final factorial

double final_factorial = 1.0;

for(i = 0; i < NTASKS; i++)

{

final_factorial *= task_params_list[i].partial_result;

}

// print final factorial

printf("Final factorial: %e\n", final_factorial);

}

return;

}

Void clockHandler1(UArg arg)

{

Task_yield();                   // To force a task switch if needed

}